## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(4108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is Double Earthing? Where it is used? |  |
| b) | Give classification of AC Distribution system based on number of wires. |  |
| c) | List the type & sizes of conductors used in rural electrification. |  |
| d) | State any 3 factors affecting earth resistance. |  |
| e) | Where will you use following types of guys & why: (i) Wind Guy (ii) Strut guy. |  |
| f) | List the different faults occurring in overhead distribution system. |  |
| g) | What is guy insulator? Where is it used? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare between 3, 3 wire & 3, 4 wire distribution systems. |  |
| b) | What is guard wire? Where it is used? Explain different types of guard wires used. |  |
| c) | A 1 km long overhead distribution line of 415V, 3, 50 Hz is to be erected along a straight route from 100KVA, 11/0.433 KV pole mounted substation. The line is to be laid with 6/1 x 3.00mm ACSR conductor weighing 171kg/Km on RCC poles of 9m length. The span between adjacent poles is 50m. Make a list of materials required. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain ABC system for rural electrification. State its advantages & disadvantages. |  |
| b) | A 1 AC distributor AB is 300m long & is fed from end A. The distributor is loaded as below: - 100 A at 0.707 p.f. lagging at 200m from A - 200A at 0.8 p.f. lagging at B. The load resistance & reactance of distributor is 0.2 & 0.1 per Km. Calculate the total voltage drop in the distributor. |  |
| c) | Write a short note on points to be considered for selecting a conductor for feeder & distributor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the below mentioned faults & its effect on distribution system. i) Lightening ii) L-L fault |  |
| b) | A classroom is to be wired using PVC casing-capping & consists of following points. i) Light points = 5, 60W each ii)Fan points = 1, 60W each iii)Plug points = 2, 1000W each. Draw the wiring plan for above & estimate the material required. |  |
| c) | Discuss 5 advantages, 5 disadvatages & applications of PVC casing–capping wiring system. | ………2/- |

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | State any 6 important considerations regarding motor installation wiring. |  |
| b) | Two, 3, 400V, 50Hz, scquirrel cage I.M. rated at 15HP & 10 HP are to be installed in a workshop. Determine the size of cable required, rating of main switch & motor switch required. Draw a single line diagram for the same. |  |
| c) | State any 6 rules relevant to domestic wiring. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Briefly explain the different methods to reduce earth resistance. |  |
| b) | Draw a neat labelled sketch for pipe earthing. |  |
| c) | Write a short note on MCB. |  |
| d) | Define Demand factor, Diversity factor, Connected load & Maximum Demand. |  |

***********