## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Control Engineering(4219)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the mathematical equation and Laplace transform form of a Parabolic signal. |  |
| b) | Different between open loop and closed loop control system. |  |
| c) | State the Hurwitz stability criterion. |  |
| d) | Draw the schematic diagram of a DC motor and show the representation of its transfer function in block diagram form. |  |
| e) | Explain when a system is said to be absolutely stable and conditionally stable. |  |
| f) | Explain any three rules of block diagram reduction. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give an example of a closed loop automatic control system. Draw the block diagram and explain the working. |  |
| b) | Explain the working of a Synchro transmitter receiver used in position control. |  |
| c) | Describe the construction of a solenoid and explain its working. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the schematic diagram of the following and find their transfer function: (i) Potentiometer (ii) Tachometer. |  |
| b) | Explain the working of a stepper motor with the help of a diagram. |  |
| c) | Find the Laplace inverse of the following: |  |
|  | (i) x(s)=1s(s+10)(ii) x(s)=40s2(s+1) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain three idea element of translational mechanical system with help of symbols and conventions. |  |
| b) | For the system shown in fig. 4(b) obtain (i) open loop transfer function (ii) closed loop transfer function (iii) characteristic equation. |  |
| c) | Obtain the steady state error of a type 0 system to standard test signal inputs. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | By means of Routh stability criterion determine the stability of the system represented by the following equation s4 + 2s3 + 8s2 + 4s + 3 = 0. |  |
| b) | Sketch the typical polar plots for the following transfer functions: (i) G(jw)=11+jwT1 (ii) G(jw)=1(1+jwT1)(1+jwT2) |  |
|  | (iii) G(jw)=1jw(1+jwT1)(1+jwT2) | …….2/- |

-2-

| c) | Describe four standard test signal used in analysis of control systems. Also represent the signals graphically and mathematically. |  |
| --- | --- | --- |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For a control system with G(s)=64(s+2)s(s+0.5) |  |
|  | Draw the bode plot and determine the gain margin and phase margin. |  |
| b) | Draw the Nyquist plot for G(s)H(s)=(s+2)(s+1)(s−1). Comment on the stability. |  |
| c) | Sketch the frequency response of a second order system to a unit step input. Comment on the influence of the damping factor value. |  |