## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Automobile Engg.

Subject: Machine Tools & Production Process-II(4060)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the operation of “gang milling” with the help of sketch. |  |
| b) | List the various types of Planers. |  |
| c) | What are the safety precautions to be followed while working on the Slotter? |  |
| d) | Write note on “Quick return mechanism” of Shaper. |  |
| e) | List the various methods of supporting boring bar in Boring machine & sketch any one. |  |
| f) | How the size of the Pillar drilling machine specified? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the various operations carried out on the Planer & explain them in brief. |  |
| b) | Explain step by step procedure of machining round shape in to hexagonal shape on Milling machine by using straddle milling operation. |  |
| c) | How a cylindrical surface can be machined on Slotter? Explain. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the difference between “UP milling” & “Down milling”? Explain with the help of sketches. |  |
| b) | List the various cutter holding devices on Milling machine & explain any two methods. |  |
| c) | Sketch any two types of Milling cutter & state their uses. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of horizontal Boring machine & explain its construction. |  |
| b) | List the operations carried out on the Drilling machine. Compare “counter boring” & “counter sinking” operations. |  |
| c) | List the different methods used for holding the tool in Drilling machine & sketch any two. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the various operations carried out on the Shaper & explain any two in brief. |  |
| b) | What do you understand by following terms with respect to Shaper: i) Adjusting stroke length ii) Positioning of stroke. |  |
| c) | Calculate the time required for taking a complete cut on a plate 400mm x 600mm, if the cutting speed is 10 m/min. The return time to cutting time ratio is 2:3 & the feed is 3 mm. The clearance at each end is 60mm. |  |
| Q.No.6. | Write short note on any four: | 4 x 3 = 12 |
| a) | Jig boring machine |  |
| b) | Multi-spindle Drilling machine |  |
| c) | Classification of Slotting machine |  |
| d) | Planner mechanism |  |
| e) | Attachments used on Milling machine |  |