## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(4026)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data, if required and state it clearly.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Derive an expression for slope at free end and deflection at free end for a cantilever beam loaded with udl of ‘w’ over its entire length. |  |
| b) | Define equivalent transformed section. Explain with examples. (any two) |  |
| c) | With neat sketches, state the effective lengths for columns for any five end conditions. |  |
| d) | A point in a strained material is subjected to two mutually perpendicular tensile stresses of 120N/mm2 and 80 N/mm2. Using Mohrs circle, locate the plane across which the resultant stress has maximum obliquity. |  |
| e) | Derive an expression for BM for a fixed beam carrying two point loads of W each, spaced equidistantly over entire span. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A beam is formed by joining two bars one of brass and other of steel, each of size 60mm wide and 15mm deep. Determine the maximum load the beam can support at centre over a simply supported span of 2m. Assume that the two bars are firmly secured to each other throughout the length with brass bar placed over steel bar. Take Esteel = 2.6 Ebrass , fs = 110 N/mm2 and fb = 70N/mm2. |  |
| b) | A rectangular column of size 1.6m x 1.2m is subjected to a compressive load of 500KN at an eccentricity of 200mm above X-X axis and 250mm to the left of Y-Y axis. Calculate stress intensities at all four corners of the column. |  |
| c) | A hollow circular column has external diameter of 350mm and wall thickness of 40mm. It carries a vertical load of 150KN at the outer edge of the column. Calculate the maximum and minimum intensities of stress in the column section. Also calculate the limit of eccentricity for this column for no tension condition. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A material is subjected to the normal stresses of 85N/mm2 on XX axis and 60N/mm2 on YY axis both tensile. In addition to this, a shear stress of 30N/mm2 acts across the both planes. Determine the principal stresses and maximum shear stress using Mohr circle method. |  |
| b) | At a point in strained material the major and minor principal stresses are 150N/mm2 (tensile) and 80N/mm2 (compressive) respectively. Find the planes on which the stress is wholly shear. Also calculate magnitude of the shear stress. |  |
| c) | A structural member is subjected to normal tensile stress of 175 N/mm2 and 90 N/mm2 in two mutually perpendicular planes. Determine the magnitude of normal, tangential and resultant stress on a plane inclined at an angle of 30 with minor stress. |  |

………2/-

-2-

| Q.No.4. | Answer any one of the following Questions: | (12) |
| --- | --- | --- |
| a) | A simply supported beam 250mm wide x 500mm deep is 7m long. Beam is loaded with an udl of 35 KN/m over its entire length. In addition to this, it caries two concentrated loads of 45KN and 30KN at a distance of 2m and 4m from left support respectively. Using Macaulay’s method, calculate maximum slope and maximum deflection of the beam. Take E= 1.5 x 105 N/mm2. |  |
| b) | A fixed beam AB is 6m long. It carries a part udl of 30KN/m for a length of 2.5m from left support. It also carries two concentrated loads of 30KN and 40KN acting at a distance of 2.5m and 4.0m from left support. Compute reactions and bending moments and plot SFD and BMD. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | Using moment distribution method calculate reactions and bending moments for a three span continuous beam ABCD for following data: - Support A is fixed and support D is simply supported. - Span AB is 3.5m with M.I. of I and is loaded with an udl of 30KN/m and carries point load of 40KN at centre. - Span BC is 4.5m long with an udl of 35 KN/m over entire span. MI of span BC is 1.5I. - Span CD is 5m long with udl of 35KN/m over entire length and at point load of 35 KN at a distance of 2m from support C. Plot BMD AND SFD. |  |
| b) | A continuous beam ABC is 8.2m long with support A simply supported and support C fixed. Span AB is 3.7m long with MI of 1.5I and carries an udl of 40KN/m over entire 3.7m length. Span BC is 4.5m long and has M.I. of 2I. It carries an udl of 25KN/m over entire span of 4.5m and two point loads of 50KN each spaced equally in the span of 4.5m. Calculate support reactions and bending moment. Plot BMD and SFD for the beam. Use moment distribution method. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | An I-section used as a column, consists of two flanges of 250mm x 20mm and web of 200 x 15mm. Column is 4.8m long with both ends fixed. Compute Rankines safe load on column assuming crushing stress of 325 N/mm2 and Rankines constant of 1/7200. Take a factor of safety of 3.0. |  |
| b) | Find the ratio of crippling load by Eulers method and Rankines method for a hollow circular column of 300mm external diameter and 200mm internal diameter. Column is 3.8m long with one end fixed and other end hinged. Take E = 2 x 105 N/mm2, crushing stress = 320N/mm2 and Rankines constant of 1/7500. |  |

***********