## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mining Engg.

Subject: Mining Technology(4292)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is an explosive? How does it function to break a rock mass? |  |
| b) | Explain the salient features of slurry explosive. |  |
| c) | What is the principle of action of roof bolting and its advantages over other support system? |  |
| d) | Describe with a neat sketch freezing method of shaft sinking. |  |
| e) | What are the various tools used by the shot firer while blasting? |  |
| f) | What are the modern methods of fast drivages of tunnels and road in coal as well as soft rock like sandstone? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is secondary blasting? What are the various methods of secondary blasting? |  |
| b) | What are the precautionary measures to be taken while transport of explosive from magazine to mine site? |  |
| c) | What is muffled blasting? Under what circumstances muffled blasting is used in a mine? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various problem associated with blasting? What care needs to take to minimize the same? |  |
| b) | What is misfire in blasting? What are the causes of misfire? What are the precautionary measures to be taken to avoid the misfire in blasting? |  |
| c) | 25 Nos of vertical holes of 100mm diameter are drilled on a hard laterite bench of 6mts height with spacing and burden of 3m x 3m respectively. A slurry explosive of 1.5g/c.c density is charged in all the holes with required stemming. Find the powder factor of the explosives. Consider sub-grade drilling and assume other data if required. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the special methods of shaft sinking? Under what conditions they are used? Explain with the neat sketch Forced Drop Shat sinking method. |  |
| b) | Describe with neat sketch Cementation method of Shaft sinking method. |  |
| c) | Describe with the neat sketch shaft centering arrangement while shaft sinking. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Describe a suitable prop withdrawer and the manner of withdrawing timber support in a depillaring district. |  |
| b) | What are the various factors which affects the selection of support system in a mine? At junction of main roadway which support you will select? |  |
| c) | Describe with the neat sketch the method of raising with Alimack Raise climber. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain two compartment method of raising. |  |
| b) | What is Blown out shot? |  |
| c) | What is pulse infusion Blasting? |  |
| d) | What is non-els? |  |

***********