## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Automobile Engg.

Subject: Hydraulic Machinery(4053)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the principle of working of a hydraulic jack. |  |
| b) | State three applications of gear pump. |  |
| c) | State the functions of air vessels in reciprocating pump. |  |
| d) | State the different types of impellers used in centrifugal pump. Give their applications. |  |
| e) | Define i) Dynamic viscosity ii) Kinematic viscosity. |  |
| f) | With the help of a suitable example, explain how fluid pressure is measured by using a inverted manometer. |  |
| g) | Define i) Steady flow ii) Uniform flow. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A differential manometer connected at the two points A and B in a pipe containing an oil of specific gravity of 0.9, shows a difference in mercury levels as 150 mm. Find the difference in pressure at the two points in bar. |  |
| b) | An isosceles triangular plate of base 3m and altitude 3m is immersed vertically in an oil of specific gravity 0.8. The base of the plate coincides with the free surface of oil. Determine i) Total pressure on the plate ii) Position of centre of pressure. |  |
| c) | A horizontal venturimeter with inlet diameter 200mm and throat diameter 100mm is employed to measure the flow of water. The reading of the differential manometer connected to the inlet and throat is 180mm of mercury. If the co-efficient of discharge is 0.98, determine the rate of flow. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The head of water over an orifice of diameter 100mm is 12m. The water coming out from the orifice is collected in a rectangular tank 2m x 0.9m. The rise of water level in this tank is 1.2m in 30 seconds. Find the coefficient of discharge. |  |
| b) | During an experiment in a laboratory, 0.05m3 of water flowing over a right angled notch was collected in one minute. If the head of the sill is 50mm, calculate the co-efficient of discharge of the notch. |  |
| c) | With the help of a neat sketch, explain the construction and working of a vane pump. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A horizontal pipe carries water at the rate of 0.04m3/s. Its diameter which is 300mm, reduces abruptly to 150mm. Calculate the pressure loss across the contraction. Take the co-efficient of contraction = 0.62. |  |
| b) | With the help of a neat sketch, explain the construction and working of a submersible pump. |  |
| c) | A single acting reciprocating pump, running at 50 r.p.m., delivers 0.00736 m3/s of water. The diameter of the piston is 200mm and stroke length 300mm. Determine the percentage slip of the pump. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of a neat sketch, explain the construction and working of a hydraulic intensifier. |  |
| b) | With the help of a suitable example, explain the hydraulic gradient line and total energy line. |  |
| c) | What are the different losses that take place in the working of a centrifugal pump? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Define i) Specific gravity ii) Compressibility. |  |
| b) | Define i) Total pressure ii) Centre of pressure. |  |
| c) | State and explain Bernoulli’s theorem. |  |
| d) | Draw a neat labelled sketch of a reciprocating pump. |  |

***********