## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg.

Subject: Generation & Transmission(4114)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (i) Connected load (ii) Demand factor (iii) Utilization factor. |  |
| b) | Give the function of Superheater and economiser in a steam power plant. |  |
| c) | Discuss the methods of starting stand-by diesel generator set. |  |
| d) | What do you mean by stay insulator? What is its function? |  |
| e) | Discuss RCC poles as line supports. |  |
| f) | State the factors on which skin effect in a conductor depends. Why skin effect does not occur for DC currents? |  |
| g) | What do you mean by stringing of conductors? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The load on a 150 MW power plant on a typical day is as follows: Time(hrs); 12-5a.m.; 5-9a.m.; 9-6p.m.; 6-10p.m.; 10-12p.m.. Load(MW); 30; 50; 90; 110; 30. - Plot the load curve. - Calculate capacity factor, utilization factor and load factor. |  |
| b) | Discuss the factors to be considered while deciding the number of units for a power plant and preparing the operating schedule. |  |
| c) | Write a short note on solar power generation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat diagram explain the working of a nuclear power plant. |  |
| b) | Discuss the main components of a diesel generator set. |  |
| c) | State and explain any six causes of failure of insulators. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is string efficiency? Discuss the methods to improve string efficiency. |  |
| b) | State the merits and demerits of EHV transmission system. |  |
| c) | Calculate loop reactance per Km of a single phase line comprising of two parallel conductors 1mt apart and 1cm in diameter, when the material of conductor is (i) Copper (ii) steel of relative permeability 50. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain with a neat phasor diagram how characteristics of medium transmission line can be determined using end condenser method. |  |
| b) | State and explain factors affecting corona. |  |
| c) | A 3 phase line delivers 4000KW at a p.f. of 0.8 lagging to the load. If sending end voltage is 22 KV, determine (i) Receiving end voltage, (ii) line current (iii) line losses (iv) line efficiency. The resistance and reactance of each conductor is 0.25/Km and 0.28/Km respectively. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief methods employed to reduce vibrations in overhead transmission lines. |  |
| b) | Write a short note on: i) Sag template ii) Capacitance of transmission line. |  |
| c) | A transmission line conductor having a diameter of 18cm weighs 0.80Kg/m. The span is 250m. The wind pressure is 35 Kg/m2 of projected area. The line is coated with ice of thickness 10mm. Ultimate strength of conductor is 7858Kg. Calculate the maximum sag if factor of safety is 2 and ice weighs 910Kg/m3. |  |

***********