## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EC

Subject: Communication Systems-II(4144)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief the principle of microwave ducting. |  |
| b) | Define i) Skip distance ii) Virtual height. |  |
| c) | Explain the primary constants of a transmission line. |  |
| d) | Define the following terms: i) Radiation resistance ii) Directivity. |  |
| e) | Compare Hertz antenna with Marconi antenna. |  |
| f) | State and explain Shannon-Hartley theorem. |  |
| g) | Why is multiplexing used in communication? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two points on earth are 1000 Kms apart and are to communicate using an HF link. If the critical frequency is 8 MHz, calculate the MUF, given that the height of ionospheric layer is 350 Kms. |  |
| b) | - Derive the relationship between VSWR and reflection coefficient. ii) A 200 ohms load is required to be matched to a 300 ohms transmission line. Calculate the characteristic impedance of the transmission line to be used. |  |
| c) | Draw a neat diagram of a parabolic reflector with cassegrain feed and explain its working. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat diagram explain the basic TE mode of propagation in a rectangular waveguide. |  |
| b) | Draw a neat diagram and explain the working of a PAM generator. |  |
| c) | Explain in detail why companding is necessary in PCM. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat block diagram and explain the working of a communication system based on delta modulation. |  |
| b) | Explain the following terms: i) Flat top sampling ii) Aperture effect. |  |
| c) | With the help of a neat block diagram, explain the working of a FSK demodulator. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat block diagram and explain the working of a QPSK modulator. |  |
| b) | Draw a neat block diagram of a communication system using FDM and explain its working. |  |
| c) | Briefly explain the following: i) Bandwidth requirement in PCM ii) On off keying. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Troposcatter link b) Hertz antenna V/s Marconi antenna |  |
| c) | Applications of Waveguides d) TDM e) Losses in antenna |  |

***********