## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Drawing & Calculations(4346)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Attempt any five from the

remaining questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Longitudinal framing system |  |
| b) | Floodable length and subdivision |  |
| c) | Stability at large angle of heel |  |
| d) | Metacentric height & Metacentric radius. |  |
| Q.No.2. | With neat sketch explain how statical stability curve is obtained from KM cross curves of stability. | (12) |
| Q.No.3. | Answer the following: |  |
| a) | With neat sketch explain how the spacing between the two bulkheads are decided using floodable length curve. | (06) |
| b) | A box shaped vessel 65m x 10m x 6m is floating upright on an even keel at 4m draught in salt water. GM=0.6m. Calculate the dynamical stability to 20º heel. | (06) |
| Q.No.4. | Construct a curve of statical stability for M.V. ‘Cargo Carrier’ when the displacement is 28000t and KG is 10m. From the curve find: | (12) |
|  | i)Range of stability ii) Maximum GZ iii) Angle of heel at which maximum GZ occurs iv) Approximate GM initial v) Righting moment at 50º heel. Refer fig.1. |  |
| Q.No.5. | Answer the following: |  |
| a) | Sketch a typical statical stability curve for unstable equilibrium and describe the information obtained from it. | (06) |
| b) | With neat sketch derive the equation to find GZ for existing centre of gravity from GZ cross curves of stability. | (06) |
| Q.No.6. | Answer the following: |  |
| a) | A box shaped vessel 150m long, 20m beam is floating upright in salt water at draught of 6m F and A. The collision bulkhead is situated 8m from the forward. Find the new draughts if the vessel is now bilged forward of the collision bulkhead. | (08) |
| b) | With neat sketch derive the equation to find the change in draught aft and forward when the ship is trim by “t”m by stern. | (04) |
| Q.No.7. | Answer the following: |  |
| a) | Sketch and describe corrugated bulkhead. | (08) |
| b) | Draw a neat sketch to show how an upper end and lower end of corrugated bulkhead is connected to ship structure. | (04) |
|  |  | …..2/- |