## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg.

Subject: Principles of Refrigeration(5046)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Refrigeration and state at least four methods of refrigeration. |  |
| b) | State the limitations of Reversed carnot cycle. |  |
| c) | Define: i) Tonne of refrigeration. ii) Coefficient of Performance. |  |
| d) | Difference between Primary and Secondary refrigerants. Give two examples for each type. |  |
| e) | State the various desirable thermodynamic properties of Primary refrigerant. |  |
| f) | Differentiate between flooded type evaporators and Dry type evaporators. |  |
| g) | List the various control systems used in Refrigeration system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat block diagram explain Vapour compression Refrigeration system. Write the functions of each component. |  |
| b) | What is Defrosting? What are the effects of frost on the refrigeration system? List the various methods of Defrosting. |  |
| c) | A vapour compression refrigerator uses methyl chloride (R-40) and operates between temperature limits of -10C and 45C. The refrigerant is dry at the entry to the compressor and acquires a temperature of 60C. Find i) Theoretical C.O.P. ii) Carnot C.O.P. Use the following properties of the Refrigerant. Saturation Temperature in C; Enthalpy in KJ/Kg. Liquid (hf); Vapour (hg). -10; 45.4; 460.7. 45; 133.0; 483.6. |  |
|  | Specific heat of vapour Cpv = 1.09 KJ/Kg-K |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between Hermatically sealed compressor and an open type compressor used in refrigeration system. |  |
| b) | With a neat sketch explain the construction and working of shell and tube type water cooled condensor. |  |
| c) | With a neat sketch, explain the working principle of thermostatic expansion valve. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat block diagram, explain Basic absorption refrigeration system. |  |
| b) | Compare Vapour compression refrigeration system with Vapour absorption system. |  |

………2/-

-2-

| c) | Write the chemical formulae and determine the designation number for the following refrigerants: - Dichloro – difluoro – methane - Dichloro – triflouro - ethane |  |
| --- | --- | --- |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | What is the necessity of refrigeration control systems? |  |
| b) | What are the probable causes of high pressure in the refrigeration system and what are its effects on the system? |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat block diagram, explain the manufacture of ice in Ice plants. |  |
| b) | Discuss the role of refrigeration in food preservation. |  |
| c) | Write a note on low temperature applications. |  |

***********