## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT/M

Subject: Hydraulics(4025)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Specific mass ii) Specific gravity iii) Specific weight. |  |
| b) | Differentiate between Newtonian fluids and Non Newtonian fluids. |  |
| c) | Define Manometer. What are its limitations? |  |
| d) | Distinguish between i) Uniform flow and Steady flow ii) Compressible flow and Incompressible flow. |  |
| e) | What is priming of Centrifugal pump? |  |
| f) | What is mean by ‘Hydraulic Jump’ in an open channel? |  |
| g) | Differentiate between Hydraulic Gradient line and Energy Gradient line. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A circular door of 1m diameter closes on an opening in the vertical side of a bulkhead, which contains water. The centre of opening is at a depth of 2m from the water level. Determine i) Total pressure on the door ii) Centre of pressure. Take specific gravity of sea water as 1.03. |  |
| b) | A tank with a square base 1.5m x 1.5m is filled with water upto a height of 0.25 m and liquid of specific gravity 0.85 above it, upto a height 0.75 m. Calculate using pressure diagram total pressure on the side of tank. |  |
| c) | A half hexagonal shape channel is to be constructed with minimum excavation & with a horizontal bottom. The maximum discharge of 20.5 m3/s of water flows through it. Determine the dimensions of the cross section if the slope of the channel bottom is 1 in 2500. Take chezy’s constant C=60. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Water is coming out of an orifice of diameter 100mm under a head of 10m. It is collected in a circular tank of diameter 1.5m. The rise of water level in this tank is 1.0m in 25 seconds. Also the co-ordinates of a point on the jet, measured from Vena contracta are 4.3m horizontal and 0.5m vertical. Find i) Co-efficient of velocity ii) Co-efficient of discharge and iii) Co-efficient of contraction. |  |
| b) | Find the bed slope of a trapezoidal channel of bed width 5m, depth of water 3.5m and side slope of 2 horizontal to 3 vertical, when the discharge through the channel is 30 m3/s. Take Manning’s N=0.03 in Manning’s formula C=1/N m1/6. |  |
| c) | A 40m long weir is divided into 10 equal bays by vertical posts, each 0.5m wide. Using Francis formula, calculate the discharge over the weir, if the head over the sill is 1.2m and velocity of approach is 2 m/s. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine difference in the elevation or free surface level between water surfaces in the two tanks which are connected by a horizontal pipe of diameter 300mm and length 500mm. Discharge rate is 300 l/s. Consider all loses in pipe. Take f= 0.008 |  |
| b) | A submarine fitted with a pitot tube moves horizontally in sea. Its axis is 12m below the surface of water. The pitot tube fixed in front of the submarine and along its axis is connected to two limbs of a U-tube containing mercury, the reading of which is found to be 200mm. Find the speed of the submarine. Take specific gravity of sea water as 1.025. |  |
| c) | A trapezoidal channel 2m wide at the bottom and 1m deep has side slopes 1:1. Determine: i) Total pressure and ii) Centre of pressure on the vertical gate closing the channel when it is full of water. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Water is flowing through a pipe having diameters 600mm and 400mm at the bottom and upper end respectively. The intensity of pressure at the bottom end is 350 KN/m2 and the pressure at upper end is 100 KN/m2. Determine the difference in datum head if the rate of flow through pipe is 60 litres/sec. |  |
| b) | Three pipes of diameters 300mm, 200mm and 400mm and lengths 450m, 255m and 315m respectively are connected in series. The difference in water surface levels in two tanks is 18m. Determine rate of flow if co-efficients of friction are 0.0075, 0.0078 and 0.0072 respectively considering major losses only. (Neglect minor losses) |  |
| c) | Differentiate between centrifugal pump and Reciprocating pump. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | How does a Cippoletti’s weir differ from a rectangular sharp crested weir? |  |
| b) | Write a short note on Re-entrant mouthpiece. |  |
| c) | Explain the principle of working of an inverted U-tube manometer. |  |
| d) | Define open channel. Classify open channels giving examples. |  |

***********