## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Estimation & Costing(4242)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) All dimensions in figures in mm, unless otherwise mentioned.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give three important purposes of estimation. |  |
| b) | Explain in short the functions of an estimation department. |  |
| c) | State the use of Measurement Page and Abstract page and draft the same. |  |
| d) | Define the following terms in Machining: i) Cutting Speed ii) Feed iii) Depth of cut. |  |
| e) | Explain in short any three press work operations. |  |
| f) | Define the following terms in Estimation: i) Detailed Estimate ii) Work Charged Establishment iii) Specifications. |  |
| g) | State the advantages and disadvantages of ‘Item Rate Contract’. |  |
| Q.No.2. | Prepare a Measurement page and Abstract page for a Gussetted Base Plate connection for a steel stanchion used in Industrial structure, for the following items of work: | (12) |
|  | i) Structural steel ii) Welding. |  |
|  | Use suitable market rates. Refer Fig1. |  |
| Q.No.3. | Give the estimation of the machined job, shown in Fig.2, made up of 60-40 Brass material, from a 40mm diameter raw round stock. (Assume suitable data) | (12) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate the cost of Fabricating an open water tank of structural steel of size 1200mm x 1400mm x 800mm height, of plate thickness 5.0 mm. All joints were fillet welds, done by using MMAW and the welder was paid Rs. 500 per 8 hrs. shift. The electrodes used were of 3.15mm  and cost Rs.140/- for 100 electrodes. The electric energy charge was Rs.5 per unit. (Assume suitable additional data) |  |
| b) | Explain with a neat sketch the use of Break Even theory for equipment replacement. |  |
| c) | Explain any three Sheet Metal operations and explain their estimation. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A Lathe cost Rs.75,000/- and have an estimated useful life of 7 years. The scrap value of the lathe is Rs.15,000/-. Calculate the depreciated amount after 5 years by Diminishing Balance method. Also give the Book value of the Lathe after 3 years. |  |
| b) | What is depreciation? Explain its importance in costing of a product. |  |
| c) | The variable cost of a certain product of a company is Rs.35/- but its fixed cost is Rs.70/-, when the production size is 500 units per month. The sales cost of the product is Rs.125/-. Calculate the increase in profit of the company if the production is raised to 750 units per month. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Explain briefly the following in contracts: - Breach of contract - Arbitration - Mode of Payment. | (6) |
| b) | i) State the importance of Measurement Book. | (3) |
|  | ii) State the action taken when whole of Security Deposit is forfeited. | (3) |