## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg./A

Subject: Machine Drawing-I(4033)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw the projections of a line 80mm long inclined at 30º to H.P. and its top view appears to be inclined at 60º to V.P. One of the ends of the line is 45mm above H.P. and 60mm in front of V.P. Draw the projections by the auxiliary plane method. |  |
| b) | A rectangular lamina ABCD of side 60mm x 40mm has its side AB in H.P. and inclined at 60º to V.P. and the plane of the lamina inclined at 60º to the H.P. Draw the projections using auxiliary plane method. |  |
| c) | Fig.1 shows two views of an object drawn by third angle method of projection. Draw the auxiliary top view in the direction of arrow ‘X’. |  |
| Q.No.2. | Answer the following Questions: | (10) |
| a) | A rectangular square prism, of base 30mm edge and 60mm height, rests on its base on H.P. such that its vertical faces are equally inclined to V.P. It has a horizontal circular hole of 30mm diameter drilled centrally through it such that the axis of the hole cuts both the diagonally opposite vertical edges. Develop the lateral surface of the prism. |  |
|  | ### OR |  |
| b) | A pentagonal pyramid with the edge of base 30mm and axis 60mm rests on its base with an edge of the base parallel to the V.P. and near the V.P. It is cut by a section plane perpendicular to the V.P., inclined at 60º to the H.P. and passing through a point on its axis, 15mm above the base. Draw the front view and the development of the lateral surface of the pyramid in one piece. |  |
| Q.No.3. | Answer the following Questions: | (12) |
| a) | A right circular cylinder, diameter of base 60mm and height 90mm, is resting on its base on H.P. It is intersected by another cylinder of 45mm diameter such that the axis of the two cylinders are mutually perpendicular and are 6mm apart. Assume the axis of the vertical cylinder to be towards the observer compared to the axis of the horizontal cylinder. Draw the projections of the solid showing curves of intersection. Assume any suitable length of the penetrating cylinder. |  |
|  | ### OR |  |
| b) | A vertical cone, base 70mm diameter and axis 80mm long is penetrated by a cylinder of 45mm diameter such that the axis of the cylinder is parallel to the axis of the cone. Draw the projections showing the curves of intersection when the axis of the vertical cylinder is 8mm from the axis of the cone and the plane containing the two axis is perpendicular to the H.P. and inclined at 45º to V.P. |  |

………2/-

-2-

| Q.No.4. | Fig.2. shows the front view and top view in 1st angle method of projection. Draw according to 3rd angle method of projection: - Sectional front view along AA - Sectional top view along BB iii) Left hand side view. | (16) |
| --- | --- | --- |
| Q.No.5. | Answer the following Questions: |  |
| a) | With the help of a suitable freehand sketch, show the following types of sections: - Full section 2) Removed section 3) Offset section - Revolved section. | (08) |
|  | ### OR |  |
| a) | Give conventional representation for the following: - Packing material 2) Wood 3) Brass and Rubber - Ball bearing. | (08) |
| b) | Draw by a double line method a 25mm diameter pipe showing: - Gate valve 2) Reducing elbow 3) Elbow 45º. | (09) |
| Q.No.6. | The object shown in fig.3 is to be fabricated by welding at different pieces A, B, C, D, E & F in the proper relative positions. Draw the orthographic view of the object and indicate the proper welding symbols. | (08) |
|  | ### OR |  |
| Q.No.6. | Show by means of sketches, the method of showing location, symbol, size and depth of the following forms of weld: - Single V-butt weld - Single bevel butt weld - All round fillet weld. | (08) |