## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(5036)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the requirements of a good steering system. |  |
| b) | What is meant by sulphation of a lead–acid battery? How does it occur? |  |
| c) | State the differences between MPFI and TBI systems. |  |
| d) | What are the advantages of synchromesh gear box over constant mesh gear box? |  |
| e) | Draw a neat flow diagram of car air conditioner and indicate the location of its components in a car. |  |
| f) | Name different types of spanners used in a garage and state their applications. What is the application of a ring compressor? |  |
| g) | State the provisions of motor vehicle Act 1988 under section 44. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a monocoque construction chassis and state its advantages over the framed chassis. |  |
| b) | Draw a neat labelled sketch of the steering linkage for a rigid axle suspension. Explain how wheel alignment is done. |  |
| c) | Draw a neat sketch of the Wishbone suspension system and explain. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the electronic fuel injection system in an MPFI engine. |  |
| b) | Explain how fuel injection timing is set in a diesel engine having individual pump fuel injection system. |  |
| c) | Name the various sensors used in MPFI system. State their functions. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of an overdrive. |  |
| b) | With the help of a neat sketch, explain the construction and working of a fluid coupling. State its advantages over a friction clutch. |  |
| c) | With the help of a neat sketch, explain the construction and working of a synchromesh gear box. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the construction and working of a Bendix drive. |  |
| b) | Draw a neat circuit diagram of a cut-out circuit and explain. |  |
| c) | State and explain the different standards by which automotive lead-acid batteries are rated. |  |

………2/-

-2-

| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Briefly explain the construction and working of a compressor of an automobile air conditioner. |  |
| b) | What is meant by eco-friendly refrigerant? |  |
| c) | State the provisions of motor vehicle Act 1988 under section 56. |  |
| d) | How do you adjust the ignition timing of a petrol engine by using a stroboscope? |  |
| e) | Write a brief report on history of automobiles. |  |

***********