## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./E/EI/COMP/EC/MED/E&EE

Subject: Digital Electronics-I(4134)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Convert the following into Binary: (0.683)10, (6E)16, (375)8 |  |
| b) | With examples explain Gray code. |  |
| c) | Draw the logic diagram and write the truth table of D flip-flop. |  |
| d) | Prove De’ Morgan laws with examples. |  |
| e) | Compare sequential and combinational Digital system. |  |
| f) | Define any three characteristics of Logic families. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram, explain the working of Master- Slave JK flip-flop. |  |
| b) | Simplify the following using K-maps - F = ABC+ABC+ABC+ABC - F (w, x, y, z) = ∑ (0,1,2,5,8,9,10,13) |  |
| c) | Prove NAND as an universal gate. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of clocked S-R flip-flop. |  |
| b) | For the following, draw the logic circuit and write the truth table. Also, explain its working. i) T flip flop ii) JK flip flop. |  |
| c) | Draw the logic symbol and write the truth table along with the equation for the following logic gates: i) NOR ii) NOT iii) EX-OR. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With necessary diagram and waveforms explain the working of Ring Counter. |  |
| b) | Explain the working of Decade Counter. |  |
| c) | With a neat diagram, explain the working of a 4–Bit Serial-In Serial-Out shift register. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With necessary diagram, explain the working of TTL Nand gate. |  |
| b) | Explain the working of DTL logic family with necessary diagram. |  |
| c) | Explain CMOS Nand gate and CMOS NOR gate with necessary diagrams. |  |
| Q.No.6. | Answer any three: (draw diagrams wherever necessary) | 3 x 4 = 12 |
| a) | Explain positive and Negative Logic. |  |
| b) | Compare synchronous and Asynchronous Counters. |  |
| c) | Write a short note on PIPO register. |  |
| d) | Write a short note on RTL. |  |
| e) | Compare different logic families. |  |

***********