## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Welding In Shipconstruction-II(4337)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the essential requirements of good welding jig? |  |
| b) | Discuss the weldability considerations in welding Austenitic stainless steels. |  |
| c) | List down the advantages & disadvantages of TIG welding. |  |
| d) | Explain the various stages of Weld Inspection & Testing. |  |
| e) | List down the principles of a sound welding design. |  |
| f) | What are the reasons for estimating welding costs? Also classify the various welding costs. |  |
| g) | List down the applications of metal sprayed coatings. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the process of MIG welding with a neat sketch. |  |
| b) | As per the Indian Register of shipping explain the Welding Procedure Qualification test. |  |
| c) | Explain the following w.r.t. dissimilar welding: (i) Thermal expansion (ii) Galvanic corrosion (iii) Weld dilution. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on welding of Austenitic stainless steels. |  |
| b) | Explain the all weld metal tensile test of a welded specimen. Also mention its significance. |  |
| c) | Discuss the X-ray radiography w.r.t a welded structure. Also mention its significance w.r.t. the shipbuilding Industry. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A lap joint is made by a plate 60 mm wide carrying a load of 10,000N which is welded to another similar plate with 4 equal fillet welds. Find the necessary size of each fillet. Assume an allowable stress of 80N/mm2 in the side fillets & 110N/mm2 in the end fillets. |  |
| b) | Calculate the total cost of a manual flux shielded metal arc welding using the data below. Weld length = 2.5 m Welding speed = 15 m/hour Operating factor = 35% Labour & Overhead charges = Rs. 35 per hour Electrode consumption = 0.35 Kg/m Electrode price = Rs. 30 per Kg Arc voltage = 22 Volts Arc Current = 200 Amps E = 0.6 Rate per KWhr = Rs. 1.2 |  |
| c) | Explain the cavity welding technique w.r.t. underwater welding process. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Discuss the process of reconditioning of worn out shaft by a metal spraying process. |  |
| b) | What are the safety precautions to be undertaken while performing an arc welding operation? |  |
| c) | Explain in brief the various types of edge preparations in welding. |  |
| Q.No.6. | Write short notes any three: | 3 x 4 = 12 |
| a) | Welders Performance Qualification |  |
| b) | Welding of High Carbon steel |  |
| c) | Ultrasonic test |  |
| d) | Costing procedure for Gas Welding |  |

***********