## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Instrumentation-I(S-4214)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State advantages of iron core inductor transducer over air core inductive transducer. |  |
| b) | Discuss the original conditioning techniques of Capacitive transducer systems. |  |
| c) | State one advantage and one disadvantage of wire wound potentiometers. |  |
| d) | A Pt 100 RTD was used to measure the temperature of a gas. The resistance of the RTD was found to be 115 ohms. Find the temperature of the gas.  = 0.00392 ohms/Celcius |  |
| e) | Explain the principle of working of thermo electric transducer. |  |
| f) | Give an example of a contact type of transducer and explain the working. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain the objectives of an instrumentation system. |  |
| b) | Compare Analog and Digital Instrumentation system. |  |
| c) | Explain the following terms: i) Primary Calibration ii) Direct Calibration iii) Routine Calibration. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and characteristics of Potentiometer. Describe the different type of Potentiometers. |  |
| b) | Explain the principle of strain gauge. Explain why and how dummy gauges are used. |  |
| c) | Explain the construction and working of RTD. Why is platinum the most widely used material in RTDs. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a block diagram explain the working of a strain measurement system. |  |
| b) | Explain the working of the following: i) Bourdon tube ii) Flapper nozzle iii) Canti-lever. |  |
| c) | Explain the principle of working of LDR. Explain any one application of LDR with the help of block diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of an inductive transducer working on the principle of eddy currents, with the help of a diagram. |  |
| b) | Draw the block diagram of a measurement system using inductive transducer and explain the working. |  |
| c) | Explain the construction, working and characteristic of LVDT. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the principle and working of three types of Capacitive transducers. |  |
| b) | Explain the application of capacitive transducer in the measurement of level of a non conducting liquid with the help of a diagram. |  |
| c) | With the help of suitable signal conditioning and data presentation blocks explain the working of displacement measurement system using capacitive transducer. |  |

***********