## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Materials & Structures(4232)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define tensile stress, tensile strain and Hooke’s law. |  |
| b) | Define Young’s modulus, Bulk Modulus and Rigidity modulus. |  |
| c) | Prove that stress due to sudden load is twice that due to gradual load. |  |
| d) | Define moment of Inertia and state the expressions for Rectangular section and Hollow circular section about centroidal XX and YY axis. |  |
| e) | A mild steel rod of 20mm diameter and 400mm long is subjected to bending due to load of 10KN at centre when tested. Calculate its bending stress. |  |
| f) | A cantilever beam of span 2m carry UDL of 10KN/m throughout. Sketch BMD of beam and its maximum bending moment magnitude. |  |
| g) | A rectangular beam of 15x25cm in section is subjected to shear force of 40KN. Find its maximum shear stress. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A bar of 2m in length is subjected to pull such that maximum stress is 150N/mm2. The bar is 25mm diameter for 1.5m of its length and 40x40mm in section for the remaining 0.5m length. Determine the pull and total elongation produced. Take E = 1x105 N/mm2. |  |
| b) | A reinforced cement concrete column of 500mm diameter consists of 8 steel bars of 25mm diameter. The columns carry a load of 1200KN. Determine the stresses produced in steel and concrete. Also load shared by both materials. Take ES = 16 EC. |  |
| c) | A metal bar 60x60 mm in cross section is subjected to axial compression of 450KN. The contraction of a 200mm gauge length is found to be 0.5mm and increase in thickness is 0.045mm. Find the values of modulus of elasticity and Poison’s ratio and Rigidity modulus. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A mild steel rod of 40mm diameter and 600mm long is subjected to load of 60KN axially. Determine stress, deformation and strain energy when the load is i) gradual ii) sudden. Also find percentage increase in strain energy due to sudden load. Take E = 2x105 N/mm2 |  |
| b) | A 2.8m long bar of 1200 mm2 cross sectional area hangs vertically and has a collar fixed at its lower end. Find the stress induced when a weight of 5 KN falls from a height of 150mm on to the collar. Also find strain energy stored in the bar. E = 120Gpa |  |
| c) | Explain the stress-strain diagram with a neat sketch and define all significant points on it. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cantilever beam of 2m span carry a UDL of 20 KN/m for 1.5m length from fixed end and a point load of 4KN at free end. Draw SFD and BMD. |  |
| b) | A beam ABC of length 8m has simple supports at A and B for a span of 6m while BC is overhang. Point loads of 40 KN and 20KN acts at 2m and 4m from A and a point load of 10KN at free end C. Draw SFD and BMD and final position and magnitude of maximum BM. |  |
| c) | A simply supported beam AB of 5m span carry a UDL of 40KN/m for 3m length starting from support A and a point load of 10KN at 1m from end B. Draw SFD and BMD and find position and magnitude of maximum bending moment. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine Moment of inertia of a I section about its horizontal centroidal axis with size of top flange 150x20mm, web 15x 250mm and bottom flange 300x40mm. |  |
| b) | Find the safe concentrated load that can be applied at the free end of 3 metre long cantilever beam. Section of beam is hollow with outer diameter = 80mm and inner diameter is 60mm. The safe bending stress for the material is 70N/mm2. |  |
| c) | A channel section beam with flanges (top and bottom) as 150x10mm and web 10x280mm is simply supported for a span of 5m. The beam carries an UDL of 35 KN/m throughout span. Determine maximum bending stress produced. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A channel section beam has symmetrical flanges of 125x10mm and web 10x280mm. Beam section is subjected to maximum shear force of 140KN. Determine the shear stress distribution across the section. |  |
| b) | A water main of 1400mm internal diameter and 16mm thickness is running full and it is simply supported over a span of 16 metres. Find the bending stress produced in the pipe material due to self weight of pipe and weight of water. Take density of pipe material 78.5 KN/m3 and that of water 10KN/m3. |  |

***********