## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Electronic Circuits(4135)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between fT and f of CE amplifier. |  |
| b) | State the applications where you prefer using transformer coupling. |  |
| c) | What is Barkhausen criteria for sustained oscillations? |  |
| d) | Draw the circuit diagram of class A pushpull amplifier and state its features. |  |
| e) | Draw characteristics of DIAC. |  |
| f) | What are the different types of multivibrators? |  |
| g) | Define transconductance and amplification factor in case of FET. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat circuit diagram of a direct-coupled amplifier using BJT. Also explain its operation and state one advantage and one disadvantage of this coupling. |  |
| b) | Sketch the input and output characteristics of a BJT in common emitter configuration and explain how h-parameters can be determined from the characteristics. |  |
| c) | For a transistor configured in common base mode, calculate Zi, Zo, Av, Ai. Given hib=1.6 K, hrb= 2 x10–4, hfb =110, hob=20mhos. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat circuit diagram explain operation of RC coupled amplifier. Also sketch and explain its frequency response. |  |
| b) | Explain how device capacitance plays dominant role in CE amplifier high frequency region. |  |
| c) | Draw the hybrid parametric equivalent circuit of a transistor configured in common collector mode and explain the significance of each hybrid parameter shown in the circuit. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Single ended amplifier is not much used in practical circuits. Instead push pull amplifier is used. Why? |  |
| b) | Draw the circuit of a class B push pull amplifier and explain its operation. What is crossover distortion? How it is minimized? |  |
| c) | Draw the circuit showing FET as VVR and explain the working with the help of waveforms. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw and explain the common source FET amplifier circuit. State the relevant voltage gain and important expressions. |  |
| b) | Explain the working of Bistable Multivibrator with a neat circuit & necessary waveforms. |  |
| c) | With necessary circuit & waveforms, explain the operation of a sine to square wave converter using two transistors. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An amplifier with negative feedback has a voltage gain of 100. It is found that without feedback an input signal of 50mv is required to produce a given output, whereas with feedback input signal must be 0.6V for the same output. Calculate A & . |  |
| b) | Obtain an expression to show that by using negative feedback the input impedance increases by a factor (1 + A). |  |
| c) | What do you understand by the term negative feedback? What are the different types of it? |  |
| d) | Explain the construction, equivalent circuit and working of UJT. |  |

***********