## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg.

Subject: Testing & Maintenance of Electrical Equipments(5104)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is meant by type test, routine test and acceptance test of electrical equipment? |  |
| b) | What is necessity of polarity test on a transformer? Explain the polarity test briefly. |  |
| c) | List the methods of measurement of temperature rise of transformer winding. |  |
| d) | Classify and explain briefly the various losses that take place in an induction motor. |  |
| e) | Enlist any six type tests to be conducted on an induction motor as per IS. |  |
| f) | Distinguish between preventive maintenance & breakdown maintenance practices. |  |
| g) | What is the purpose of conducting sustained 3 short circuit test on a 3 synchronous machine? State any two of its features. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain the experimental setup and procedure for determination of efficiency of a DC machine using the Hopkinson’s test. |  |
| b) | Distinguish between Brake test & Swinburne’s test performed on a DC machine. (Write atleast 6 points of distinction) |  |
| c) | A 100KW, 500V shunt generator was run as a motor on no load at its rated voltage and speed. The total input current drawn was 9.5 A including shunt field current of 2.5 A. The armature circuit resistance is 0.1. Calculate the generator efficiency when operating at full load & ½ full load. Draw suitable circuit diagram. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain the experimental setup & procedure of open circuit & short circuit test on a 1 transformer for determination of its losses and efficiency. |  |
| b) | A 10KVA, 200/400V, 50HZ, 1 transformer gave the following test results: O.C test 200V 1.3 A 120W S.C test 22V 30A 200W Find the parameters of equivalent circuit as referred to the primary i.e LV winding. Draw suitable equivalent circuit. |  |
| c) | With neat diagram explain ratio test on a 1 transformer. Also state its significance. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A 500V, 500HZ, 6 pole, 3 squirrel cage induction motor running at 975 rpm draws an input power of 40KW. The stator losses are 1 KW and friction & windage loss is 2 KW. Calculate: i) Slip ii) rotor cu loss iii) motor output in BHP & iv) efficiency. |  |
| b) | With a neat diagram explain the HV testing of an Induction motor. |  |
| c) | Calculate full load efficiency of a 3, 415V, 50HZ delta connected induction motor from the following test data: On no load: Power intake =1500W at rated input voltage On full load: Line current is 50A, p.f. is 0.85, slip =0.04, Resistance of stator winding per phase = 0.5. Assume the ratio of stator core loss to friction and windage loss is 3:2. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the typical maintenance schedule to be followed for a power transformer up to 1000KVA capacity as per IS. |  |
| b) | What is “drying out process” of an electrical rotating machine? Why and when is it necessary? State various steps involved in ‘drying out’ of a motor. |  |
| c) | Explain the need of revarnishing of windings of an electrical equipment. State methods for revarnishing. Briefly explain anyone of the methods. |  |
| Q.No.6. | Write notes on any two: | 2 x 6 = 12 |
| a) | Power frequency voltage withstand test on power transformer |  |
| b) | Sudden 3 short circuit test on 3 synchronous machine |  |
| c) | Acceptance tests, its significance & types |  |

***********