## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical & Electronics Engg.

Subject: Power Generation, Transmission & Distribution(5091)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the advantages of grid system. |  |
| b) | Give general specification of DG set. |  |
| c) | What is post type insulator? Give any one application of the same. |  |
| d) | State the specifications required for procurement of pin insulator. |  |
| e) | Compare AC three wire and four wire distribution system in terms of volume of conductors and its applications. |  |
| f) | What is cleat wiring? State its applications. |  |
| g) | List any three I.E. rules for earthing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss important points to be taken into consideration while selecting the size and numbers of units for a given load curve. |  |
| b) | Explain merits and demerits of hydroelectric power plant over thermal power plant. |  |
| c) | A generating station has following daily load cycle Time (Hrs) 0-6 6-10 10-12 12-16 16-20 20-24 Load (MW) 40 50 60 50 70 40 Draw the load curve and find i) maximum demand ii) units generated per day iii) averages load iv) load factor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are bundled conductors? Give the advantages of bundled conductors? |  |
| b) | What do you mean by the term ‘string efficiency’? Explain in brief methods to improve string efficiency. |  |
| c) | Sate the merits and demerits of HVDC system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on (i) RCC poles (ii) Steel towers. |  |
| b) | What is corona? What are its effects on transmission lines? What are the methods of reduction of corona loss? |  |
| c) | Compare radial, ring and interconnected system on the basis of load, reliability of supply and economy. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive the expression for cross sectional area and volume of conductor required in DC 2 wire distribution system. Give applications of the same. |  |
| b) | Briefly explain system voltage levels and permissible voltage drop in various parts of urban and rural distribution system. |  |
| c) | Compare PVC casing capping and conduit wiring. (any 8 points) What are the advantages of concealed conduit wiring? |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A small classroom of 6m x 6m x 3.5m is to be provided with 3 fan points and 4 tube points. Draw a neat layout of wiring indicating the location of switchboard, light and fan points. Also estimate the material required for PVC casing capping wiring for this installation. |  |
| b) | i) With a neat diagram explain staircase wiring. ii) List the methods to improve earth resistance. |  |
| c) | Write a neat sketch, explain pipe earthing and state its advantages over plate earthing. |  |

***********