## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EI/EC/MED/COMP/E&EE

Subject: Digital Electronics-II(4138)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Define ‘Resolution’, ‘Accuracy’ and ‘Conversion time’ of an A to D converter. | (3) |
| b) | What is the use of level amplifier in a digital to Analog converter? | (1) |
| c) | Write the formula for time period of an Astable multivibrator using IC555. | (1) |
| d) | Mention the different types of memories. | (2) |
| e) | What is a Nibble Multiplexer? | (1) |
| f) | Draw the diagram of Invert Matrix of a PLA. | (2) |
| g) | Draw the circuit diagram of a Full adder. | (2) |
| h) | Write the expression for “Carry” output of a BCD adder. | (1) |
| i) | How is binary multiplication carried out? Give one example. | (2) |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any one method of Digital to Analog conversion, with a neat circuit diagram for a 3-bit system. Write the expression for output voltage. |  |
| b) | Explain simultaneous method of Analog to Digital conversion. Draw the circuit diagram for 8-bit conversion. What are the advantages and disadvantages of this method? |  |
| c) | Explain continuous method of A to D conversion with a neat block diagram. Mention the advantages and disadvantages. |  |
| Q.No.3. | Explain with a neat diagram and truth table, the working of any two of the following: | 2 x 6 = 12 |
| a) | 16 to 1 Multiplexer |  |
| b) | BCD to Decimal Decoder |  |
| c) | Priority Encoder |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat circuit diagram explain the working of IC555 as a monostable multivibrator. Write the expression for time period. |  |
| b) | What is contact bounce problem? Draw the diagram of any one method of contact debouncing circuit and explain it. |  |
| c) | Explain the working of a 8 bit parallel adder with a neat circuit diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagram showing the connections for obtaining 8K x 8 bit memory from 4K x 4 bit memory IC and explain it. |  |
| b) | Explain the working of an Adder-Subtractor with a neat diagram. |  |
| c) | Draw and explain ROM array using diode switches. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain with an example the working of ROM as PLD. Draw the diagram and truth table. |  |
| b) | Explain AND matrix of a PLA with a neat diagram. |  |
| c) | - Write a note on FPGA devices. - Write the steps involved in programming a PLA. |  |

***********