## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Substation Practice(5105)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List any six factors to be considered for selection of site for substation. |  |
| b) | What is a bus coupler? With a neat diagram explain its use. |  |
| c) | What do you mean by CT Burden. What will happen if CT secondary is open when its primary is carrying current? |  |
| d) | What is an Earth Switch? State its importance in substation. |  |
| e) | State different colour lamps used in control panel. What each colour will indicate? |  |
| f) | State function of following in HVDC substation: i) Convertor valves ii) AC Filter Banks. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain single sectionalised bus bar scheme. State its advantages & disadvantages. |  |
| b) | With a neat diagram explain construction & working of clip-on Ammeter. |  |
| c) | With a neat diagram explain different types of transformer substations. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw symbols and state functions of: - Lightning Arrester - Line trap - Isolator. |  |
| b) | State and explain different type of power factor tariffs. |  |
| c) | The load on an installation is 800KW, 0.8 p.f. which works for 3000 hrs per annum. The tariff is Rs. 200 per KVA plus Rs. 2 per unit. If the p.f. is improved to 0.9 lagging by means of a loss free capacitor costing Rs. 100 per KVAR, calculate the annual saving effected. Allow 10% per annum for interest & depreciation on capacitor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain different methods of neutral earthing. |  |
| b) | Explain Metal oxide lightning arrester & its working. |  |
| c) | What is surge absorber? Briefly explain any two types of surge absorbers. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat diagram explain different laying method for cables. State its advantages & disadvantages. |  |
| b) | State and explain factors on which current carrying capacity of a cable depends. |  |
| c) | Determine the size of cable required to carry maximum load current of 30 A. The length of cable is 400m. The allowable voltage drop is 6% of rated voltage. The supply is single phase 230V, 50Hz. Following cables are available. Size of Cable. No & dia in mm; Normal area mm2; Current Rating Ampere; Approx. Ampere- meter per volt drop. 19/1.12; 19.35; 62; 1050. 19/1.32; 25.80; 74; 1475. 19/1.625; 38.70; 97; 2200. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Battery charging methods |  |
| b) | Testing of transformer oil & measures to improve its quality |  |
| c) | Line clearance & safety precautions in carrying out work in substation |  |

***********