## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg.

Subject: Illumination Engineering(5106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the qualities of good lighting. |  |
| b) | State the type of lighting scheme adopted for a textile industry and justify your choice. |  |
| c) | State the law of illumination applicable for measurement of illumination level at the wall of a room, in case the lighting source is located exactly at the centre of the ceiling. Hence give its statement. |  |
| d) | Define the following: M.H.C.P., M.S.C.P. and M.H.S.C.P. |  |
| e) | Define the following: Direct ratio (ii) space to mounting height ratio. Also, state the range of their nominal values. |  |
| f) | Classify the streets. Also state the basis for their classification. |  |
| g) | Classify the flood light projectors. Also state the basis for their classification. |  |
| h) | Give at least three special features of hospital lighting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the factors affecting good lighting. Also, explain each of these factors in brief. |  |
| b) | State the different types of lighting systems. Also state one application of each of these lighting systems. |  |
| c) | Can the stroboscopic effect be eliminated? Give your choice. Also state the methods for its elimination, if any. Explain any one of such method used with the help of neat diagram. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the significance of polar curves? State their types. Also state the steps for plotting polar curves. |  |
| b) | Define the following terms: (i) Lumen (ii) Candle power (iii) Lux (iv) Efficacy of lamp. |  |
| c) | Calculate the illumination level 2m below the centre of 3m2 circular shaped luminance ceiling which emits 10,000 lumens, given that luminous intensity = Lumens/. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the factors affecting the selection of maintenance factor. Also, state the criteria for selection of reflection factors of ceiling, wall and floor in case of indoor lighting design. Also, specify the % reflection factors for them. |  |
| b) | State the most appropriate method used in interior lighting design. Give all the steps involved in drawing the layout of luminaires in a plan. Also, state the types of mounting arrangement of luminaires indicating the type of interior for which each type of arrangement is used. |  |

………2/-

-2-

| c) | An illumination level of 100Lux is required in a hall of 40m x 8m. The lamps are required to be placed at a height of 3 metres above the work bench. Assume utilization factor of 0.6, lamp efficacy as 14 & candle depreciation as 18%. Estimate the number of lamps, wattage of each lamp. Also, assume suitable space to mounting height ratio. |  |
| --- | --- | --- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the different arrangements in street lighting and explain each in brief with the help of neat sketches. |  |
| b) | Define outreach and overhang. How is overhang determined? State the factors to be considered for selection of mounting height in street lighting luminaires. State the nominal values of mounting height recommended for Group A and Group B roads. |  |
| c) | Give four features of monument and statue lighting. |  |
| d) | Give four features of Auditorium lighting. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is aiming of flood light projectors? State the different methods of aiming of flood light projectors. |  |
| b) | - State the objectives of flood lighting. - What are the various types of lamps used in flood lighting? - Define beam spread. - State the conditions under which different types of flood light projectors are used. |  |
| c) | A vertical red wall of 25m x 10m (height) of a monument is to be illuminated from a distance of 30m by projectors planted on ground. Find the number of fittings or lamps and their arrangement given that the beam factor as 0.5, beam spread is 25C (maximum), maintenance factor as 1.6, waste light factor is to be assumed appropriately. Also, specify the lighting source chosen. |  |

***********