## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg.

Subject: Principles of Airconditioning(5047)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following Psychrometric properties: i) Wet bulb temperature ii) Specific humidity iii) Humid Specific heat. |  |
| b) | List three heat sources which contribute to System Heat Gain. |  |
| c) | What are the advantages of removal of impurities present in the air? |  |
| d) | Define i)) Throw ii) Drop iii) Spread as referred to Air distribution system. |  |
| e) | What are the factors to be considered for selection and location of outlets? |  |
| f) | List the desirable properties of an Ideal insulating material. |  |
| g) | What is Economical thickness of Insulation? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A sample of moist air is at 30 DBT and 60% RH under a total pressure of 1.013 bar. Determine i) Dew point temperature ii) Specific humidity iii) Specific enthalpy of the moist air, using steam table and emperical formulae. |  |
| b) | With the help of sketch explain the construction and working of ‘Sling Psychrometer’. |  |
| c) | A circular duct of diameter 0.50 metre is to be replaced with a rectangular duct having an aspect ratio of 1:2, determine the cross section of rectangular duct assuming the constant velocity for both the ducts. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following psychrometric processes and represent the process on the psychrometric chart (i) Heating and humidifying (ii) Cooling and dehumidifying. |  |
| b) | With the help of neat block diagram explain “Winter Air conditioning system”. Represent the process on Psychrometric chart. |  |
| c) | 36 Kg/s of air stream at 15C DBT and 13C WBT are mixed with another stream of air 14 Kg/s at 25C DBT and 18C WBT. Determine DBT and WBT of the resulting mixture. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch, explain the construction and working of ‘Window Air Conditioner’. |  |
| b) | Differentiate between All air system and All water system of air conditioning. |  |
| c) | Discuss the special features of Air conditioning system used in computer centers. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain, how the following heat sources are accounted for cooling load estimation: (i) Occupancy load (ii) Infiltration load (iii) Solar heat gain. |  |
| b) | An air-conditioned space is maintained at 24C DBT and 50% RH, when the outdoor conditions are 35C DBT and 28C WBT. The space has a sensible heat gain of 18 KW and the air is supplied to the space at 8C saturated condition. Determine (i) The mass flow rate of air supplied in Kg/s (ii) Latent heat load in the room (iii) Cooling load of the A/c plant in TR if 15% of total mass of the air supplied to the space is fresh air and remaining is recirculated. |  |
| c) | With a neat sketch, explain the construction and working of any one type of humidifier. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch, the construction and working of self cleaning viscous filter. |  |
| b) | With a neat sketch, explain two position pneumatic control system. |  |
| c) | Explain Face and by-pass control system. |  |

***********