## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the qualities of an Ideal Estimator. |  |
| b) | Differentiate between approximate and detailed estimate. |  |
| c) | What is importance of provision of contingencies and work charged establishment? |  |
| d) | Give 3 different units of measurement for any 3 items of work as per IS:1200. |  |
| e) | Define terms scrap value, salvage value, market value. |  |
| f) | Explain opening of tender in PWD offices. |  |
| g) | List various conditions of contract to govern nature of work to be carried out. |  |
| h) | Explain clearly need of preparing revised estimate. |  |
| Q.No.2. | Answer any one of the following Questions: | 1 x 15 = 15 |
| a) | Fig No.1 show plan and wall section of a single storey building. Calculate quantities of any five items. (Assume any missing data) - Earthwork in excavation - P.C.C. (1:3:6) in foundation and plinth - R.C.C in lintel and slab including Reinforcement - Wood work in door & window frames - Internal colour washing - External plaster 15mm thick. |  |
| b) | Determine the quantities of earthwork for the portion of a road between chainages 50 and 60 from the following data, length being measured with a standard 20m chain. (Assume suitable data if required) |  |
| Chainage; 50; 51; 52; 53; 54; 55; 56; 57; 58; 59; 60. Ground level; 131.1; 131.2; 130.90; 131.2; 130.8; 130.7; 130.6; 130.4; 129.1; 129.5; 129.7. |
|  | The formation level at chainage is 130.0 and the road is in the rising gradient of 1 in 200. The width of formation is 10m and side slopes 1.5:1 in embankment and 1:1 in cutting. The lateral slope of the ground may be assumed as level. Also calculate cost of earthwork in banking and cutting at prevailing rates. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Prepare rate analysis for brick masonry work in superstructures using 1:3 CM. Quantity = 10m3 |  |
| b) | Prepare rate analysis for 18mm thick plaster in CM 1:4, Quantity = 10m2. |  |
| c) | Briefly explain conditions of termination of a contract. |  |
| d) | Write a detailed specification for 6mm thick cement plaster in ceiling. |  |

………2/-

-2-

| Q.No.4. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Draft a notice inviting tender for the work of construction of Government Health centre to be executed by PWD in 2 years at an estimated cost of 80 lakhs. |  |
| b) | Write short note on duties and responsibilities of Divisional officer and Superintending officer. |  |
| c) | Explain importance of maintaining site and office records. |  |
| d) | Define approximate estimate and enumerate its purpose. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Explain two methods of taking out quantities in buildings. |  |
| b) | Differentiate between freehold property and leasehold property. |  |
| c) | Explain piecework agreement and show piecework agreement format. |  |
| d) | Explain the types of tenders. |  |