## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A 55mm diameter nozzle delivers a jet of water at 0.1m3/s perpendicular on a vertical plate that moves at 10m/s away from the jet. Find the force exerted by the jet. |  |
| b) | What is a Draft tube? Why is it used in reaction turbine? |  |
| c) | What may be the probable causes if the centrifugal pump fails to deliver any water? |  |
| d) | Define Slip and Negative slip in a reciprocating pump. |  |
| e) | Draw a neat sketch of any one type of vacuum pump. |  |
| f) | Give symbols for the following pneumatic components: - Spring return single acting cylinder - Shuttle valve - 2/2 direction control valve. |  |
| g) | Draw a Bleed-off hydraulic circuit using symbols. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Pelton wheel develops 12,000 KW at shaft under a head of 380m. The speed of the wheel is 750 r.p.m. and overall efficiency is 86%. Find the wheel diameter; number of jets and diameter of each jet. Given: Jet ratio, m=10; speed ratio Ku=0.46; Cv=0.98. |  |
| b) | Explain the working of Kaplan turbine with the help of a neat sketch. |  |
| c) | Write a short note on constant efficiency curves of turbine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A centrifugal pump delivers 1500 lit/s when working against a head of 8m. The impeller is running at 200 r.p.m. The diameter of impeller is 1.22m and area at outer periphery is 0.65m2. If the Vanes are set back at an angle of 30 at the outlet, find i) Manometric efficiency ii) Work done by the impeller. |  |
| b) | Give the classifications of centrifugal pumps and explain the different types of impellers used with the help of sketches. |  |
| c) | What do you mean by ‘priming’ of a pump? What are the different arrangements employed for priming centrifugal pumps. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A double acting reciprocating pump having piston diameter 360mm has a stroke of 300mm. The pump is discharging at the rate of 2.4m3 water per minute at 45 r.p.m. through a total height of 10m. Find the % slip of the pump and work done by the pump. Neglect area of piston rod. |  |

………2/-

-2-

| b) | Draw indicator diagram with acceleration and friction head for reciprocating pump in the following cases: - Pump without air vessel - Pump with air vessel fitted close to the cylinder. |  |
| --- | --- | --- |
| c) | Distinguish between centrifugal and reciprocating pump. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Francis turbine of inlet diameter 1.2m operates under a head of 150m and requires a discharge of 6m3/s at a speed of 400 r.p.m. The guide vane angle is 20 and water leaves the runner without any whirl. Find the power and hydraulic efficiency if the runner is 0.1m wide at inlet. |  |
| b) | What is meant by ‘Cavitation’? What is the significance of Thomas cavitation factor in turbines? |  |
| c) | Write a short note on ‘Selection of turbines’. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short note on maintenance of Pneumatic Control System. |  |
| b) | Explain with the help of a neat sketch, the meter-out hydraulic circuit. Give its one application. |  |
| c) | With the help of a neat sketch explain the working of internal gear pump. |  |

***********