## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Shopfloor Practice-I(4333)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the various methods of taper turning. |  |
| b) | Which are the various types of grinding machines used in Industry? |  |
| c) | How do you specify Pillar drilling machine? |  |
| d) | Write short note on Jig boring machine. |  |
| e) | What are precautions to be observed while bending a pipe? |  |
| f) | List any three differences between brush painting and spray painting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a horizontal boring machine and name the various parts. |  |
| b) | List the various operations carried out on vertical boring machine and sketch any two. |  |
| c) | What are the various methods of mounting the tool on boring bar? Sketch them. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the various tool holding devices used on drilling machine and sketch any two. |  |
| b) | With the help of neat sketch explain the construction and working of Pillar drilling machine. |  |
| c) | What is the difference between “counter boring” and “counter sinking”? (four points) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of hand operated pipe bending machine and explain its working. |  |
| b) | A pipe of 350mm and length 500mm is required to be fabricated out of 10mm plate. Explain the procedure involved along with development of plate. |  |
| c) | Write short notes on: - Heating and bending (ii) Factors involved in bending |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the differences between three jaw self centering chuck and four jaw independent chuck? (6 points) |  |
| b) | Draw a neat sketch of lathe machine and name the various parts. |  |
| c) | What is mandrel? What is the purpose of using a mandrel? List various types of mandrel and sketch any one. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Facing operation |  |
| b) | Shapes of grinding wheels |  |
| c) | Dressing of grinding wheel |  |
| d) | Roller painting |  |
| e) | Surface preparation before painting |  |

***********