## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction to Thermodynamics(4334)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State second law of thermodynamics. |  |
| b) | What is the significance of the area under a process line on p-v diagram |  |
| c) | The camshaft speed is half that of the crankshaft in a four-stroke I.C. engine. Explain why? |  |
| d) | Explain why an I.C. engine needs cranking to start it? |  |
| e) | Define dryness fraction of steam. |  |
| f) | Define tonne of refrigeration. |  |
| g) | Convert the following readings of pressure to bar: (i) 800 KPa (ii 1,12,000 N/m2 (iii) 1,520 mm of Hg. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain why specific heat of a gas at constant pressure is always greater than its specific heat at constant volume |  |
| b) | Explain thermal equilibrium and Zeroth law of thermodynamics. |  |
| c) | A quantity of gas is expanded isothermally from initial condition of 0.1m3 and 735 KPa to a final pressure of 118 KPa. Find (i) the final volume (ii) the work done and (iii) the heat supplied during the expansion process. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In the characteristic equation pv = RT for unit mass of a gas, prove that R is the difference between the specific heats at constant pressure and at constant volume. |  |
| b) | In an engine working on the ideal Otto cycle, the temperatures at the beginning and the end of adiabatic compression are 100C and 473C respectively. Find the compression ratio and the air-standard efficiency of the engine. Take  = 1.4 for air. |  |
| c) | In an ideal Diesel cycle the temperature at the beginning and end of compression are 57C and 603C respectively, whilst those at the beginning and end of expansion are 1,950C and 870C respectively. Determine per kg of working fluid for which R= 0.287 KJ/Kg and  = 1.4, (i) the heat received in kJ (ii) the heat rejected in kJ (iii) the work done in kJ and (iv) the ideal thermal efficiency. If the compression ratio is 14:1 and the pressure at the beginning of compression is 100KPa, determine the maximum pressure in the cycle. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketches, explain the working of a four-stroke cycle petrol engine. |  |
| b) | With the help of a neat sketch, explain the water-cooling system of an I.C. engine. |  |

………2/-

-2-

| c) | The following observations were made during a trial on a single-cylinder two stroke cycle oil engine having a cylinder diameter 20cm and piston stroke 30cm: Duration of trial, 40 minutes; total number of revolutions, 12,000; indicated m.e.p., 580KPa; net load on the brake, 1,240 newtons; effective radius of the brake wheel, 0.58 metre; fuel consumption, 4.32 kg; calorific value of fuel oil, 44,000 kJ/kg. Calculate: (i) the mechanical efficiency, (ii) the indicated thermal efficiency and (iii) the brake thermal efficiency of the engine. |  |
| --- | --- | --- |
| d) | Explain the following terms as applied to steam: - Internal latent heat - External work of evaporation and - Internal energy. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the condition of steam in each of the following cases: - At a pressure of 10 bar, if 2,640 KJ/kg are required to produce it form water at 0C. - At a pressure of 8 bar and temperature 200C, and - At a pressure of 12 bar and volume 0.144 m3/kg |  |
| b) | Use the steam table to calculate the following: - The internal energy of 1kg of steam of dryness fraction 0.85, at a pressure of 6 bar. - The internal energy of 1kg of steam at 10 bar and temperature 200C. Take Kp of superheated steam as 2.1 KJ/KgK. - Internal energy of 0.125 m3 of steam at 12 bar and dryness fraction 0.75. |  |
| c) | Differentiate critically between mechanisms of heat transfer by free convection and that of forced convection. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the concept of “black body” in heat transfer. State plank’s law of emission of radiant heat. Explain its significance. |  |
| b) | List the desirable properties of a good refrigerant. |  |
| c) | With the help of a neat sketch, explain the working of a vapour compression refrigeration cycle. |  |

***********