## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Piping Fabrication & Installation(5342)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) All dimensions in the figures are in mm unless specified.

5) Refer to the data provided at the end of the Question paper.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the major phases in piping installation. |  |
| b) | Give the function of a reducer in a pipeline. Differentiate between a Concentric and Eccentric reducer. |  |
| c) | Sketch the different welded pipe flanges. |  |
| d) | Give the symbols of the following pipe fittings in flanged, welded and soldered connections in single line: i) Eccentric Reducer ii) Check valve iii) Safety valve. |  |
| e) | What is P & ID of a Piping? What does it indicate? |  |
| f) | Explain Nozzle schedule? Explain its importance. |  |
| g) | What is piping flexibility? Why is it required? |  |
| Q.No.2. | Design a pipe of material ASTM A53 GB, to fabricate a pipeline to conduct a fluid at 250C, delivery pressure of 100 psi, and discharge of 500 cc/sec. The total length of the pipeline is 752 m and in its length are present 1 - Globe valve, 5 - 90 short radius Elbows, 3 - Unions and 2 - Tees. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the Isometric view and Developed view of the portion of the pipeline whose Plan is shown in fig.1. (Draw out of scale but dimension clearly) | (8) |
| b) | Make a Bill of material of the pipeline shown in Fig.1, Q.3a. Neglect weight of pipeline and fitting. | (4) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in short the activities involved during the fabrication of a pipeline. |  |
| b) | Explain the procedure adopted to select the piping material for a certain application. |  |
| c) | Explain the importance of test certification of a pipeline. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the compression pipe bending process. |  |
| b) | Calculate the minimum length of the straight pipe required to fabricate a 4 - piece bend of bend angle 110 and internal radius of pipe bend of 150mm, the pipe O.D. is 60mm. |  |
| c) | Explain the importance of piping supports. Sketch a roller support and explain its working. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with an example the procedure to conduct Hydro –test on a pipeline. |  |
| b) | Explain the different special design considerations to be given importance while fabricating a toxic gas pipeline. |  |
| c) | Write a note on piping installation. | ………2/- |