## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./E/EI/COMP/EC/MED/E&EE

Subject: Basic Electronics(4132)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain zener and avalanche breakdown. |  |
| b) | Give two examples each of semiconductor, insulator and conductor. |  |
| c) | What do you understand by thermal runaway? |  |
| d) | Compare between CB and CE configuration. |  |
| e) | Draw the symbol and structure of N channel depletion type MOSFET and P channel JFET. |  |
| f) | Explain in brief directly and indirectly heated cathode. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain intrinsic semiconductor. |  |
| b) | Explain a half wave rectifier with the help of circuit diagram and waveform. |  |
| c) | In a centre tap full wave rectifier, the load resistance RL = 1K. Each diode has a forward bias dynamic resistance rd of 5. The voltage across half of secondary winding is 220 Sin 314t. Find (i) peak value of current (ii) dc or average value of current (iii) rms value of current (iv) ripple factor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how Zener diode act as voltage regulator. (i.e. for varying load and varying input voltage) |  |
| b) | Explain VI characteristics of PN junction diode. |  |
| c) | With the help of circuit diagram, explain capacitor filter. Draw necessary waveform. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of NPN transistor. |  |
| b) | Draw and explain Common Emitter input and output characteristic. |  |
| c) | Calculate the values of three currents in a fixed bias circuit with emitter resistor. Given Vcc = 10V, RB = 1M, RC = 2K,  = 100, RE = 1K. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the structure of N-channel JFET and explain drain characteristic. |  |
| b) | Draw and explain DE (enhancement and depletion) MOSFET. |  |
| c) | i) What is the relation between  and  of a transistor? ii) If dc = 0.97 and leakage current is 2A in a transistor and emitter current is 3mA, calculate the collector and base current. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction & working of Vacuum diode. |  |
| b) | Derive the Q point of collector to base bias circuit. What are its advantages and disadvantages? |  |
| c) | With the help of circuit diagram, explain the working of Pentode. |  |

***********