## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Circuits & Networks(4136)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) port ii) mesh iii) node. |  |
| b) | Distinguish between lumped and distributed parameters. |  |
| c) | State Kirchoff’s current law and explain it in brief. |  |
| d) | What do you mean by active and reactive power? |  |
| e) | Explain the concept of resonant frequency fr in a series RLC circuit. |  |
| f) | Define time constant with respect to a simple RL and RC circuit. |  |
| g) | Sketch the frequency response of a low pass and a high pass filter. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate the equivalent resistance, current supplied by the source and current flowing in 2 resistor in the circuit shown in fig.1. |  |
| b) | Find the current flowing through all the resistances in fig.2 using Kirchoffs laws. |  |
| c) | State Nortons theorem and explain the procedure to compute current flowing in a particular resistor using the theorem with example. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find current in the arm AB using superposition theorem in the circuit shown in fig.3. |  |
| b) | State and prove the maximum power transfer theorem. |  |
| c) | Calculate the current flowing through 12 resistor using thevenins theorem in fig.4. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on resonance in series RLC circuit. |  |
| b) | Determine total impedance, current I, phase angle  and voltage dropped across each element in the circuit shown in fig.5. |  |
| c) | A series RLC circuit consists of 50 resistance, 0.2H inductance and 10f capacitor with an applied voltage of 20 V. Calculate resonant frequency, Q factor, lower and upper cut off frequencies and bandwidth of the circuit. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the following graphical representations: - Impedance Z versus frequency - Phase angle  versus frequency. |  |
| b) | Explain the dc response of a RL circuit with neat circuit diagram, necessary equations and response sketches. |  |
| c) | With the help of circuit diagram and waveforms explain the operation of RC differentiator. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Design a t-type attenuator to provide 60dB attenuation with design impedance of 500. |  |
| b) | Design a constant K type high pass filter with cut off frequency of 1 KHz with a terminated design impedance of 800 . |  |
| c) | Write short notes on: i) Attenuators ii) Decibel and neper. |  |