## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(4339)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is Compulsory. Answer any 5 from the remaining                            questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Thrust deduction factor |  |
| b) | Frictional resistance |  |
| c) | Stern Gear |  |
| d) | Reynolds’s Number |  |
| e) | Cavitation |  |
| f) | Guldhammer and Harvald method |  |
| Q.No.2. | Answer any one of the following Questions: | (12) |
| a) | What is meant by ‘Ship Resistance’? How can it be determined from model tests? |  |
| b) | Explain the function of a stern tube of a ship. With the help of neat figure, explain a typical stern tube of a vessel. |  |
| Q.No.3. | Describe the following propeller terms: | (12) |
|  | a)Root b) Tip c)Rake d)Skew e)diameter f)Pitch. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | With reference to ship propulsion, explain how a negative apparent ship is produced. | (2) |
| b) | A ship 120 m long, 15.3m breadth, 7.16m draft has a displacement of 9800 tonnes in sea water. | (10) |
|  | The propeller has a pitch ratio of 0.9. At a propeller rotational speed of 1.8 rev/sec, the speed of the ship is 16 Knots and the real slip ratio is 0.25. Calculate |  |
|  | i) The pitch of the propeller |  |
|  | ii) The diameter of the propeller |  |
|  | iii) Wake speed |  |
|  | iv) Apparent slip ratio. |  |
|  | Taylor wake fraction=0.5CB-0.05 |  |
| Q.No.5. | A ship of length 116m, breadth 18.70m, draft 7.1m and displacement 8400 tonnes has the following values of effective power. Speed; 15.5; 16.5; 17.5; 18.5; 19.5. PE(KW); 2300; 2860; 3553; 4390; 5425. | (12) |
|  | Determine for a geometrically similar ship of displacement 9300 tonnes, the effective power at a speed of 17.4 Knots. Given: |  |
|  | i)Kinematic viscosity=1.115X10-6 m2/s |  |
|  | ii)Wetted surface area of ship of length 116m =2553 m2 |  |

………2/-

-2-

| Q.No.6. | What are the special propeller types? Explain the characteristics, features and applications of any 2 special types of propellers. | (12) |
| --- | --- | --- |
| Q.No.7. | Answer any one of the following: | (12) |
| a) | Explain in brief the construction of a reverse reduction gear box. What is its function? |  |
| b) | Propeller of diameter 5.8m provides a speed of 16 Knots to ship at 120RPM. Efficiency of the propeller is 68%. If froude wake fraction is 0.22 and thrust deduction fraction 0.15, find PE, PT, actual thrust and torque for a delivered power of 1860 KW. |  |

***********