## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED

Subject: Microprocessors(PD&I)-II(4147)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain slave mode of 8257. |  |
| b) | List 3 main features of 8259 Programmable Interrupt Controller. |  |
| c) | Explain scan section of 8279. |  |
| d) | Write Bit set/reset control word of 8255 to set PC4. |  |
| e) | Describe the write display RAM control word of 8259. |  |
| f) | Explain in brief the structure of floppy disk. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the Block diagram of 8155. |  |
| b) | Explain with timing waveforms Mode1:Output control signals of 8255. |  |
| c) | Determine the control word of 8255 to send data from Port A to Port B and from CL to CU. Also explain each bit of control word. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with block diagram the transmitter section of 8251. |  |
| b) | Draw block diagram of 8279. |  |
| c) | Describe in detail the control word of 8253. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw Block diagram and describe 8257. |  |
| b) | Explain the interrupt operation of 8259. |  |
| c) | Draw block diagram of 8259 and explain the Priority Modes. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of capacitive keys and Hall effect Keyswitches. |  |
| b) | Explain D/A conversion techniques. |  |
| c) | Explain the steps in formatting and partitioning the Hard disk. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | GPIB Interface Bus |  |
| b) | Traffic Light Controller |  |
| c) | Printers |  |

***********