## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg./P/A/SB

Subject: Strength of Materials(4032)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain Hook’s law with reference to a rod subjected to compression. |  |
| b) | Define lateral strain and Poisson’s ratio. |  |
| c) | State and explain perpendicular axes theorem with an example. |  |
| d) | Show that maximum stress intensity due to a suddenly applied load is twice the stress intensity produced by load of same magnitude applied gradually. |  |
| e) | State the assumptions made in the theory of pure bending. |  |
| f) | Explain with an example of rectangular cross section column subjected to eccentric load, the condition for stress to remain wholly compressive. |  |
| g) | State Rankine’s formula and list each of the terms involved. |  |
| h) | What is the difference between thin and thick cylinders with respect to stress induced due to internal pressure? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel bar consisting of two lengths 180mm and 220mm with diameters 10mm  and 20mm  is subjected to compressive force axially of 2 KN. Find stresses in each part and the total decrease in length. E=2x105 N/mm2 |  |
| b) | A reinforced concrete column 300mm x 300mm in section is provided with 8 bars of 20mm diameter. The column carries a load of 400 KN. Find the stresses in concrete and steel if ES=2.1x105 N/mm2 , EC=0.14x105 N/mm2. |  |
| c) | A steel rod is 20m long at a temperature of 20C. Find the free expansion when the temperature is raised to 65. Also find the temperature stress produced when the rod is permitted to expand by 5.8 mm. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | At a point the principal stresses are 140 N/mm2 and 75 N/mm2 both tensile. Find the normal and tangential stresses on a plane inclined at 60 to the axis of the major principal stress. |  |
|  | ### OR |  |
| a) | Draw “Mohr’s stress circle” for principal stresses of 80 N/mm2 tensile and 50 N/mm2 compressive and find the resultant stresses on planes making 22 and 64 with the major principal plane. Find also the normal and tangential stresses on these planes. |  |
| b) | If steel rod 50mm in diameters is 3 metres long, find the maximum instantaneous stress induced when a pull of 100 KN is suddenly applied to it. Find also the instantaneous elongation. Take E= 2x105 N/mm2 |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A cantilever AD is 1.5m long. It is fixed at A & free at D and is subjected to point loads 400N, 300N, 200N at every 0.5m from A. Draw S.F.D and B.M.D. |  |
|  | ### OR |  |
| a) | A simply supported beam 9m in span is subjected to uniformly distributed load 18 KN/m over 4m from the left support. Draw SFD and BMD. |  |
| b) | Find the moment of inertia about horizontal centroidal axis of ‘L’ section whose overall depth is 100mm and overall width 80mm and thickness is 20mm throughout. Also find the M.I. about bottom most fibre. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A pipe of external diameter 60mm and 10mm thickness and 5m long is supported at its ends. The pipe carrier a point load of 100N at its centre. Calculate the maximum bending stress induced. |  |
| b) | A short column of external diameter 150mm and internal diameter 100mm carries an eccentric load. Find the greatest eccentricity which the load can have without producing tension on the cross section of the column. |  |
| c) | A solid shaft 125mm in diameter transmits 120KW at 160 rpm. Find the maximum shear stress induced in the shaft. Find also the angle of twist in a length of 7.5m. Take C=8x104 N/mm2. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A thin shell 1m in diameter is subjected to an internal pressure of 1N/mm2. If the thickness of the shell is 10mm, find the circumferential stress, longitudinal stress and circumferential strain. Take E=2x105 N/mm2 and 1/m = 0.3. |  |
| b) | A strut 2.5 m long is 60mm in diameter. One end of the strut is fixed while other end is hinged. Using Euler’s formula, find safe compressive load for the member allowing a factor of safety of 3.5. Take E= 2x105 N/mm2 |  |
| c) | A cantilever 3m long carries a uniformly distributed load over the entire length. If the slope at the free end is 1, find the deflection at the free end. |  |

***********