## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-I(4054)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give neat sketches of following engines: i) Radial ii) Delta iii) H-type. |  |
| b) | Sketch the ideal PV & T- diagram for Dual cycle engine. Label neatly and name the various processes. |  |
| c) | Distinguish between Timed and Continuous Fuel Injection in SI Engines. |  |
| d) | What is Stoichiometric air-fuel ratio for S.I Engines? Explain. |  |
| e) | Sketch the Fractional distillation method of refining crude oil. |  |
| f) | Emphasize the necessity of cooling of IC Engines. |  |
| g) | A water-cooled IC Engine run very hot. Trouble–shoot the possible causes. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is scavenging of engines? Where is it used? Explain one method with sketches. |  |
| b) | Describe the engine valves and the gudgeon pen using neat sketches. |  |
| c) | What is cylinder liner? With sketches describe its types, material, its ability to cool rapidly and method of fitting it on the cylinder block. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the solid, liquid and gaseous fuels which can be used to run IC Engines. |  |
| b) | Using neat sketches explain the construction and working of an elementary carburator. |  |
| c) | Describe with sketch the acceleration circuit of a complete carburator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain different types of fans for engine cooling system. |  |
| b) | What are the functions of a CI engine Fuel injection system and its requirements? |  |
| c) | Using a neat sketches explain the working of fuel injection pump (BOSCH/MICO). |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give two points of difference between wet sump and dry sump lubrication. Explain the pressure feed Lubrication system. |  |
| b) | With sketches explain the combustion chambers of SI Engines. |  |
| c) | Compare between the detonation in SI and CI Engines. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | List the types of lubricants in general and their suitability for use in IC Engines. |  |
| b) | A 3-cylinder, 4-stroke S.I. engine having bore x stroke as 69mm x 71mm has a clearance volume of 32cc for each cylinder. Calculate its Compression Ratio. If the engine runs at 4000 rpm, find the volume of charge inhaled in one minute by the engine. |  |
| c) | With a sketch explain how a hydraulic dynamometer may be used to measure torque of an I.C. Engine. |  |

***********