## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Microprocessor Based Instrumentation(5114)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How is a 20 bit physical address generated in 8086? |  |
| b) | Explain stack memory and give instructions related to it. |  |
| c) | Enlist salient features of a microprocessor. How is it different from a Microcontroller? |  |
| d) | When is the overflow flag set in 8051 Microcontroller? |  |
| e) | What are the addresses of accumulator and PSW in RAM memory of 8051 Microcontroller? |  |
| f) | Explain how bit manipulation is achieved in 8051 Microcontroller. |  |
| g) | Explain how an external device can interrupt 8051 Microcontroller. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the flag register of 8086 microprocessor. |  |
| b) | Draw and explain internal block diagram of 8086 microprocessor in brief. |  |
| c) | Compare 8086, 80386 and Pentium processors with regards to i) data line ii) address line iii) Speed iv) Compatibility of instruction set. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the timer mode operation of 8051 Microcontroller. |  |
| b) | Explain each bit of TCON and TMOD registers, with the help of register formats. |  |
| c) | Explain internal RAM memory organisation of 8051 Microcontroller. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the serial data transmission modes (i) mode 0 (ii) mode 1 of 8051 Microcontroller. |  |
| b) | Explain the importance of INT0 and INT1 interrupts of 8051 Microcontroller. How can 8051 be interrupted by external device at INT0 pin? |  |
| c) | Explain the difference between following instruction: i) MOVX A, @RP and MOVX A, @DPTR ii) MOVC A, @A+PC and MOVC A, @A+DPTR. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Write a program to reverse a block of data bytes stored from internal RAM memory location 00H and size of the block of data byte is ten. |  |
| b) | Write a program to count, data bytes greater than 2EH. Ten bytes of data are stored at internal RAM of 8051 from address 40H. The result should be saved in register R0. |  |
| c) | Write set of instructions to divide a byte stored in RAM of 8051 at address 40H by a byte stored in RAM location 41H and to store the result in RAM locations 50H and 51H. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Logic and compare instructions of 8051 |  |
| b) | Applications of Microcontroller |  |
| c) | Bus interface unit of 8086 microprocessor |  |
| d) | Special function registers of 8051 Microcontroller |  |
| e) | Addressing modes in 8051 Microcontroller |  |

***********