## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(4104)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | For the circuit shown in fig.1, change the network from star to delta. |  |
| b) | For the circuit shown in fig.2, by circuit simplification technique, find the current I. |  |
| c) | A capacitor and resistance are connected in series to an a.c. supply of 200V, 50 Hz. The current is 2A and power dissipated is 80W. Calculate the value of resistance and capacitance of capacitor. |  |
| d) | Derive an expression for the current drawn by a R.C circuit connected to an a.c. supply. Draw the vector and waveform diagram. |  |
| e) | Three resistors of 100ohm each are connected in delta to a 3-phase, 50HZ, 400V supply. Calculate the total power consumed. If the same three resistors are connected in star, to the same supply, calculate the power consumed. |  |
| f) | Derive an expression for the transient voltage in a d.c., RC circuit. |  |
| g) | Explain the transients in RL circuits (D.C). |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the current in 4ohm resistance for the circuit shown in fig.3 by using superposition theorem. |  |
| b) | Find the thevenin’s equivalent circuit across terminals A & B for the circuit shown in fig.4. |  |
| c) | Using mesh method, determine the current supplied by battery for the circuit shown in fig.5. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For the circuit shown in fig.6, total current supplied is 15A. Find the current in each branch and its angle w.r.t. total current. What is the voltage? |  |
| b) | A coil of resistance 20ohm and inductance of 0.04H is connected to an a.c. supply of 23OV, 50 Hz. Calculate i) current ii) p.f. and iii) voltage drop across resistance and across inductance. |  |
| c) | A series RLC circuit consists of a 100ohm resistance, inductance of 0.318H and unknown capacitor. When connected across 230V, 50HZ a.c supply, the current is 230A. Calculate i) The value of capacitance ii) total power consumed and iii) Reactive power. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive an expression for the resonant frequency of the parallel circuit. |  |
| b) | A coil having an inductance of 50mH and resistance of 10ohm is connected in series with a 25F capacitor across 200V, a.c. supply. Calculate the i) current at resonance ii) resonant frequency iii) value of quality factor and iv) bandwidth. | …….2/- |

-2-

| c) | The inductive circuit of resistance 2ohm and inductance 0.01H is connected to 250V, 50Hz supply. What capacitance placed in parallel will produce resonance? Find the total current taken from the supply and current in each branch. |  |
| --- | --- | --- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The four arms of a Hay’s bridge are arranged as follows: AD is a coil of unknown impedance, DC is a non inductive resistance of 1000 ohm, CB is a non inductive resistance of 880 ohm in series with a standard capacitor of 1.4F, BA is non inductive resistor of 16,500 ohm. If the supply frequency is 50HZ and the bridge is balanced, find the value of inductance and resistance of the coil. |  |
| b) | Draw the circuit diagram of Wien’s bridge. Give its equations at balanced. What are its advantages and disadvantages? |  |
| c) | Find out the positive, negative and zero phase sequence components of the three unbalanced voltage vectors: VA=1030, VB=30-65 & VC =15145 V. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 3-phase, 3-wire, 240V, ABC system supplies a delta connected load in which ZAB=2590,ZBC=150 and ZCA=2030 Ohm. Find the line currents and total power. |  |
| b) | A 220V, 3-phase voltage is applied to a balanced delta connected 3-phase load of phase impedance (6+j8)ohm. - Find the line current. - What is the power consumed per phase? - What is the reactive power per phase? |  |
| c) | Three identical impedances are connected in star to a 3 phase supply of 400V, 50Hz. The line current is 20A and the total power is 10KW. Calculate the resistance and inductance of each impedance. |  |