## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engg./EC/COMP/EI/MED/E&EE

Subject: Microprocessors-I(8085)[4142]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List any three arithmetic instructions. |  |
| b) | Give two points of difference between stack and stack pointer. |  |
| c) | List any three interrupts and state their priority levels. |  |
| d) | Specify the functions of ALE signal and RDsignal. |  |
| e) | Define instruction cycle and machine cycle. |  |
| f) | Specify one instruction each which uses immediate addressing mode, register addressing mode and direct addressing mode. |  |
| g) | List any three flags used by 8085 and state their functions. |  |
| h) | Differentiate between serial and parallel communication. (Give three points of difference) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the timing diagram for the instruction MVI M, 45H. The opcode for MVI M is 36H, which is stored in memory location 6000H. Assume that contents of H register is 20H and contents of L register is 10H. |  |
| b) | Describe how instructions may be classified based on their functions. Give one example of each type. |  |
| c) | State the number of bytes, number of T-states and number of machine cycles for each of the following instructions: i) MOV A,M ii) LDAX B iii) ADD C iv) ANI 45H |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe execution of the CALL instruction by listing the machine cycles. Use a suitable example. |  |
| b) | Explain nesting of subroutines and multiple-ending subroutines with suitable programs. |  |
| c) | With a neat block diagram, explain how a seven–segment display may be interfaced to the 8085 microprocessor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the 8085 interrupt process using INTR signal. |  |
| b) | Explain the SIM and RIM instructions. |  |
| c) | Explain functions of the following signals of 8085: i)HOLD ii)READY iii)IOM iv) SID. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With flowcharts, describe the different methods to generate time delays. |  |
| b) | Design a continuous hexadecimal up-counter having a delay of 5ms between successive counts. Assume that the microprocessor clock frequency is 3MHz. Illustrate the delay calculations. |  |
| c) | Write a program to count the number of bytes which have even parity in a block of thirty bytes, which are stored starting from memory location 2500H. Store the result in register B. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write program to add n numbers stored in memory locations starting from 2101H. The value of n is stored in memory location 2100H. Store the 8-bit result in memory location 2600H. |  |
| b) | A memory block of size 2010 starts from memory location 2700H. Write a program to transfer contents of this block to another memory block starting from 2600H. |  |
| c) | Show contents of registers A, D, E and H after execution of the following program. Illustrate the register contents after each instruction. MVI A, 43H LXI D, 4525H LXI H, 3050H DAD D XCHG ADD A SUB E |  |

***********