## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg.

Subject: Electrical Engineering Materials(4101)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any three electrical and any three mechanical properties that a good conductor should possess. |  |
| b) | Give the applications of Platinum as resistive material. |  |
| c) | Explain extrinsic semiconductors. Name a few extrinsic semiconductors. |  |
| d) | What do you mean by dielectric loss in an insulating material. State any three factors that affect it. |  |
| e) | List the factors affecting the insulation resistance of a material. |  |
| f) | Define (i) Magneto motive force (ii) reluctance (iii) Co-ercive force. |  |
| g) | What are thermocouples? State their applications. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare copper and aluminium as conductors for power transmission lines. |  |
| b) | Discuss the special properties of following materials: (i) Tungsten and (ii) Brass (as conductor materials). |  |
| c) | i) State the required properties of underground cables. |  |
|  | ii) List out the properties of carbon as resistive material. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the function of copper oxide as rectifier and mention its applications. |  |
| b) | Write a brief note on any three of the following: (i) Thermistors (ii) Variastors (iii) LCD (iv) Strain gauges. |  |
| c) | Explain what do you mean by ‘Hall effect’. Give the application of Hall effect. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the process of polarization of dielectric. |  |
| b) | Explain in brief conduction through gaseous and liquid dielectrics. |  |
| c) | Explain the charging and discharging of a dielectric material. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss PVC and ceramic as insulating materials used in electrical industries. |  |
| b) | Explain in brief the thermal properties of insulating materials. |  |
| c) | Explain in brief various types of thermosetting resins. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give examples of hard magnetic materials. List their properties and applications. |  |
| b) | What do you mean by hysterisis? Explain in brief with the help of a diagram. |  |
| c) | i)Discuss the applications of radio active materials. |  |
|  | ii)Write a short note on contact materials. |  |

***********