## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-II(CE401) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define: i) Transiting of telescope ii) Departure iii) Consecutive co-ordinates. |  |
| b) | What do you understand by the terms: i) Horizontal equipment ii) Grade of contour iii) Contour interval. |  |
| c) | State commonly used radii & degree of curve for roads & railways. |  |
| d) | Give applications of GPS. |  |
| e) | State uses of Total station. |  |
| f) | Describe the principle of tacheometry with the help of diagram. |  |
| g) | What is interpolation of contour? Why is it required? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure for measurement of horizontal angle by reiteration method. Also draw observation table. |  |
| b) | Write a note on ‘Gale’s Traverse Table’. |  |
| c) | What is balancing of traverse? State various rules by which balancing can be done. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are permanent adjustments? State relationship between various axis of Theodolite. |  |
| b) | Explain with neat sketch procedure to measure vertical angle with the help of Theodolite. Draw observation table also. |  |
| c) | Explain direct method of locating contour with sketch & example. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how will you obtain in field the tacheometric constants (give proper sketch). |  |
| b) | When sighted horizontally, a tacheometer reads 1.645 & 2.840 corresponding to stadia hair on vertical staff 120 m away. Focal length of object glass is 20 cm & distance of object glass to trunnion axis is 15 cm. Calculate stadia interval. Also write distance formula. |  |
| c) | What is tacheometer? List down the features tacheometer should incorporate? Write down distance formula. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain Rankine’s method of stetting out curve. |  |
| b) | Calculate ordinates at 10 m distance for a circular curve having a long chord of 80 m & a versed sine of 4 m. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Digital Planimeter |  |
| b) | Arithmetic method of interpolation of contour |  |
| c) | Indirect methods of contouring |  |
| d) | Principle & application of digital level |  |

***********