## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Shipbuilding Engg.

Subject: Shopfloor Practice-I(4333) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the factors involved in bending? Briefly explain. |  |
| b) | Write note on brush painting. |  |
| c) | List various types of boring machines. |  |
| d) | What do you understand by term ‘abrasives’ used in grinding wheel? List types of abrasives. |  |
| e) | What are the safety precautions to be followed while painting a cabin of ship? |  |
| f) | How do you specify a lathe machine? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of hydraulic hand operated pipe bending machine and explain its working. |  |
| b) | A pipe is required to be fabricated having a dimension of 350mm outside diameter and 500mm long out of 10mm thick plate. Explain steps involved in fabricating along with developed size of plate required. |  |
| c) | With the help of sketch explain taper turning method by swiveling the compound rest. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of pillar drilling machine and name parts. |  |
| b) | Write note on: i) Gang drilling machine ii) Multiple spindle drilling machine. |  |
| c) | List the methods of holding the tools in drilling machine and briefly explain any two methods. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of sketch differentiate between steady rest and follower rest. |  |
| b) | List the properties of cutting fluid used while machining on lathe. |  |
| c) | Draw a neat sketch of horizontal boring machine and label the parts. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List and explain the operations carried out on horizontal boring machine. |  |
| b) | What are the methods used to mount the boring bar on the horizontal boring machine? Explain with simple sketches. |  |
| c) | Draw neat sketch of plate bending machine and label its parts. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Spray painting |  |
| b) | Marking system on grinding wheel |  |
| c) | Grazing and loading in grinding wheel |  |
| d) | Lathe centres |  |
| e) | Sensitive drilling machine |  |

***********