## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(4001) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write a short note on classification of survey. |  |
| b) | Draw the conventional signs for the following: i. Railway line ii. Marshy land iii. Road. |  |
| c) | How will you check the incorrect length of a chain? |  |
| d) | What are the precautions to be taken in entering the field book for a chain survey? |  |
| e) | What is cross-section levelling? |  |
| f) | Mention the specific use of Planimeter and Sextant. |  |
| g) | State the advantages of dumpy level over tilting level. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure of direct ranging. |  |
| b) | Write a note on errors due to incorrect length of chain and their corrections. |  |
| c) | Explain the selection of survey stations in chain triangulation and also define i) Baseline and ii) check line. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch the construction and use of an optical square. |  |
| b) | The following bearings were observed in a closed traverse. LINE; FB; BB. AB; S 20 30 E; N 20 30 W. BC; S 40 45 W; N 40 45 E. CD; N 30 0 W; S 30 0 E. DA; N 45 0 E; S 45 0 W. Find the included angles and do the check. |  |
| c) | The following bearings were observed in a closed compass traverse: LINE; FB; BB. AB; 140 45; 318 15. BC; 216 30; 38 00. CD; 209 15; 30 15. DE; 319 45; 139 45. EA; 60 15; 240 15. At which of these stations would local attraction be suspected? Find the corrected bearings of these lines. |  |

……2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain profile leveling. |  |
| b) | What are the reasons for local attraction and also state the precautions to be taken. |  |
| c) | The following consecutive readings were taken with a leveling instrument at an interval of 20m. 2.135, 1.760, 0.165, 3.250, 2.835, 2.050, 1.840, 0.995, 0.345, 1.620, 2.455 and 3.640. The instrument was shifted after fourth and eight reading. The first reading was taken of BM of RL 100.00m. Rule out page of level field book, enter the readings and calculate RL’s of all points. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain tilting level and also state its use. |  |
| b) | Explain the rise and fall method. Also state its advantages and disadvantages. |  |
| c) | The following consecutive readings were taken with a dumpy level. 0.894, 1.643, 2.896, 3.016, 0.954, 0.692, 0.582, 0.251, 1.532, 0.996, 2.135 The instrument was shifted after fourth and eighth readings. RL of Benchmark is 820.765m, which was taken on first point. Calculate the RL’s of the points. Rule out the page of a level field book and enter the readings. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Permanent adjustments of dumpy level |  |
| b) | Representative fraction and scales in survey work |  |
| c) | Cross-staff survey |  |
| d) | Planimeter |  |

***********