## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Architectural Engg.

Subject: Building Materials-II(AE201) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is Light weight concrete? State any two advantages of using Light weight concrete. |  |
| b) | State any three factors on which the period of curing depends. |  |
| c) | State three important factors to be considered for choosing a flooring material. |  |
| d) | What are the ingredients of a Varnish? |  |
| e) | What is Bitumen? Mention any two uses of Bitumen in building Industry. |  |
| f) | List out three important properties of Glass. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by proportioning concrete? Explain (i) Arbitrary Method (ii) Water cement ratio method. |  |
| b) | Write any six important properties of cement concrete. |  |
| c) | What is meant by mixing of materials of cement concrete? Explain any two ways of mixing. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write short notes on following: (any two) (i) Rubber flooring (ii) Gypsum (iii) Gypsum plaster. |  |
| b) | Explain the procedure of construction of Marble tile flooring. |  |
| c) | List two advantages and two disadvantages of Glass flooring. |  |
| d) | List out four uses of adhesives in building Industry. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain any two (i) Turpentine Varnishes (ii) Storm water drainage pipe (iii) Two disadvantages of G.I. pipes. |  |
| b) | List out four advantages each of Aluminium paint and Anti-corrosive paint. |  |
| c) | What is Distempering? Explain the process of Distempering. |  |
| d) | Write two examples each of thermosetting resins and thermoplastic resins with their specific use in building Industry. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare (i) Impreg timber and Compreg timber (ii) Industrial form of timber and Natural form of timber. (give three points on each) |  |
| b) | Explain in detail ‘Veneers’ and its uses in building Industry. |  |
| c) | Explain three characteristics with appropriate sketch of: (i) Lamin board (ii) Batten board. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write any four market forms of gypsum. |  |
| b) | Which are the most preferred pipes for P.W.D water supply pipeline? Explain any two. |  |
| c) | Differentiate between cutting and silvering. |  |
| d) | Explain any four varieties of Glass and state its use. |  |

***********