## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical & Electronics Engg.

### Subject: Power Generation, Transmission & Distribution(5091) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the terms: i) demand factor and ii) diversity factor. |  |
| b) | What do you mean by sag? State the factor affecting sag. |  |
| c) | Give the values of permissible voltage drop in various parts of distribution system for urban, semi-urban and rural areas. |  |
| d) | Give any three methods used to reduce earth resistance. |  |
| e) | With a neat diagram show the wiring for one lamp and one fan. |  |
| f) | State any three IE rules relevant to earthing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain in brief working of a nuclear power plant. |  |
| b) | State and explain factors to be considered while selecting the numbers and rating of generating units as per the given load curve for a power plant. |  |
| c) | Discuss the main components of diesel generator set. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the merits and demerits of HVDC transmission system. |  |
| b) | In a 33 KV overhead line, there are three units in the string of an insulator. If capacitance between each insulator pin and earth is 11% of self capacitance of each insulator, find i) the distribution of voltage over 3 insulators and ii) string efficiency. |  |
| c) | What is carona? State its advantages and disadvantages. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain any six causes of failure of insulators. |  |
| b) | Explain in brief any three types of conductors used in overhead transmission. |  |
| c) | i) Write a short note on steel tower support structure. ii) State the factors affecting earth resistance. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of feeder, distributor and service mains. |  |
| b) | Derive & compare DC-2wire and AC-single phase 2 wire system based on volume of conductor material required. |  |
| c) | Compare radial, ring and interconnected distribution system on basis of load, reliability of supply and economy. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain merits and demerits of PVC casing capping wiring. |  |
| b) | Give any six IE rules relevant to domestic wiring. |  |
| c) | i) Draw and explain staircase lighting circuit. ii) Write a short note on pipe earthing. |  |

***********