## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Materials & Processes(4238) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain what is meant by steel? Give the various types of plain carbon steels. |  |
| b) | Discuss the interstitial and substitutional types of solid solutions. |  |
| c) | List down the various groups of special alloy steels. Also explain 18-4-1 High speed steel. |  |
| d) | Explain the identification of metals and alloys by spark testing. |  |
| e) | Write short note on concept and prevention of weld decay. |  |
| f) | State the difference between cold working and hot working. |  |
| g) | Explain the various metallurgical effect of welding. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the Iron-carbon equilibrium diagram of steel up to 2% carbon and explain in brief cementite formation. |  |
| b) | Explain the formation of austenitic martensitic and ferritic types of stainless steel with their significance. |  |
| c) | Discuss the effect of carbon, silicon, manganese, sulphur and phosphorus on the properties of steel. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the difference between Grey cast iron and white cast iron. |  |
| b) | What is corrosion in welds? List down the various factors affecting corrosion. Also discuss the remedies to prevent corrosion. |  |
| c) | Explain the procedure to carry out tensile testing on mild steel specimen. Also discuss its significance. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of epitaxial and competitive growth in weld metal solidification. |  |
| b) | Discuss the effect of time and temperature of the ageing process on the mechanical properties of steel. |  |
| c) | Explain the effect quenching rate, grain size, composition of steel and alloying elements on hardenability of steel. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the process of work hardening in brief and explain the changes in properties as a result of work hardening. |  |
| b) | State the properties and application of commonly used Insulating material. |  |
| c) | Discuss the phenomenon of Age hardening in Al–Cu system with a neat sketch. |  |
|  |  | ……..2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Plastics and its applications |  |
| b) | HAZ |  |
| c) | Alloy structural steel |  |
| d) | Process of manufacture of steel |  |
| e) | Cold spinning |  |
| f) | Tempering |  |

***********