## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Production Engg.

Subject: Machine Tools & Production Process-III(4075)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Show the block diagram of saddle-type turret showing the movements of various moving elements. |  |
| b) | What are the governing parameters of lapping process? Discuss the influence of these parameters. |  |
| c) | What are single spindle automats? Give its use. |  |
| d) | Explain the need for dressing and truing of grinding wheels. |  |
| e) | Clearly indicate the differences between pull type and push type broaches. |  |
| f) | Why is the use of bright bars preferred in automatic lathes? Justify. |  |
| g) | Explain the concept of grade of grinding wheel. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the meaning of structure of a grinding wheel. What abrasive is recommended for hard, tough tool steels? Why? |  |
| b) | How a grinding wheel is marked (coded)? Describe Indian standard marking system. |  |
| c) | What is centreless grinding? Discuss the centreless grinding operation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between capstan, a turret and an engine lathe. |  |
| b) | Discuss a swiss type automatic lathe with a neat sketch. |  |
| c) | Briefly explain method of classifying automatic and semi-automatic lathes. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Which type of broaches are generally used in production? Where each of them is used? |  |
| b) | List any application of broaching and state the factors to be considers for economical broaching. |  |
| c) | Explain in detail the method of producing threads using die-heads. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is ‘Thread chasing’ & ‘Thread milling’? |  |
| b) | What is microfinishing? Explain the lapping process with a neat sketch. |  |
| c) | What is ‘Thread grinding’ & ‘Plunge cut grinding’? |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Surface broaching |  |
| b) | Internal grinding process |  |
| c) | Main constituents of grinding wheel |  |
| d) | Work holding devices for capstan & turret lathe |  |
| e) | Planning operation sequence for a given job on turret lathe |  |

***********