## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg./A

Subject: Hydraulic Machinery(ME404) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Calculate the specific weight, mass density and specific gravity of a liquid having a volume of 10m3 and weight of 50 KN. |  |
| b) | Briefly explain capillarity of water. |  |
| c) | Explain positive and negative gauge pressure with help of neat sketch. |  |
| d) | Compare laminar and turbulent flow. Give one example of each. |  |
| e) | Define Reynold’s number. State its significance. |  |
| f) | What remedial measures can be taken to avoid water hammer? |  |
| g) | What do you understand by the term slip in case of reciprocating pump? |  |
| h) | State the applications of Accumulators. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of a U-tube manometer. |  |
| b) | A simple manometer containing mercury was used to find the pressure in a pipe containing water as shown in fig.1. The right limb of the manometer was open to atmosphere. Manometer readings are given in the figure. Find the pressure in pipe in Kilo Pascals. |  |
| c) | A circular plate of 2.5m diameter is vertically immersed in water, so that its center is 4m below the water surface. Find the total pressure on the plate and the point where it acts. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular plate 2m wide and 3m deep is immersed in water in such a way that its plane makes an angle of 30 with the water surface and its upper edge is 1.5m below the water surface. Calculate the total pressure and the position of center of pressure. |  |
| b) | A Venturimeter 400mm diameter at the main and 150mm at the throat. If the difference of pressure is 250mm of mercury and the meter coefficient is 0.97, calculate the discharge of oil through the Venturimeter. Take sp. gravity of oil as 0.8. |  |
| c) | A head of water over an orifice of diameter 45mm is 15m. What is the actual discharge of the jet? Given Cv = 0.98 and Cc = 0.7. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Water is flowing through a pipe 1500m long with a velocity of 0.8 m/s. What should be the diameter of the pipe, if the loss of head due to friction is 8.7m? Take the coefficient of friction for the pipe as 0.01. |  |
| b) | With the help of neat sketch explain the construction and working of a Pelton wheel turbine. | ……2/- |
|  | -2- |  |
| c) | State the differences of Francis turbine from a Pelton wheel turbine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the power required to drive a centrifugal pump which delivers 0.05 m3/s, if the overall efficiency of the centrifugal pump is 70% and manometric head is 30m. |  |
| b) | Explain the following: i) Priming of a centrifugal pump ii) Net positive suction head. |  |
| c) | With the help of neat sketch explain the working of a single acting reciprocating pump. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following: i) Centrifugal pumps in parallel ii) Specific speed of a pump. |  |
| b) | With the help of a neat sketch explain the construction and working of a rotary vane pump. |  |
| c) | A horizontal pipe of 200 mm diameter suddenly enlarges to 300 mm diameter. After some length it suddenly reduces to 150 mm diameter. If the water flowing in the pipe is 0.05 m3/s find: i) Loss of head due to sudden enlargement ii) Loss of head due to sudden contraction. |  |