## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT

Subject: Public Health Engineering(4015) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Mention common parameters to understand water quality. |  |
| b) | What is meant by variation in demand of water? |  |
| c) | What is detention period? Explain its significant in designing a sedimentation tank. |  |
| d) | Draw a neat sketch of Gate valve. |  |
| e) | Draw a neat sketch of house service connections from the street mains. |  |
| f) | List methods of disposal of solid waste. |  |
| g) | What is significance of determining B.O.D.? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the sources of surface water and ground water? Which of these sources are suitable for: i) Rural water supply scheme ii) Urban water supply scheme. |  |
| b) | What are the requirements of wholesome water? Explain precautions to be taken against water borne diseases. |  |
| c) | What is design period in designing water supply scheme? How the population of town is forecasted? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat ketch explain process of water treatment in Rapid Gravity Sand filter? |  |
| b) | With a neat sketch explain the working of following units on treatment of water: (i) Screens (ii) Aerator (iii) Clari-flocculator. |  |
| c) | With the aid of neat sketch bring out difference between dead end and grid iron layout of distribution system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch showing internal distribution of water supply in a building with location of pipelines and position of all fixtures & fittings. |  |
| b) | With a neat sketch explain the necessity of providing manhole in a sewer line. |  |
| c) | Sketch a trickling filter and explain how it works. What are the precautions that should be taken for their effective operation and maintenance? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a typical layout of sewers in a sewerage project explain the principle governing design of such system. |  |
| b) | Explain the location, functioning and design criteria consideration for septic tank and soak pits. |  |
| c) | What are types of solid wastes? How are they collected? |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Role of Public Health Engineer |  |
| b) | Activated sludge process |  |
| c) | Types of Traps-location & functions |  |
| d) | Fire Hydrant |  |
| e) | Break Point Chlorination |  |

***********