## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Industrial Instrumentation(4218)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a sketch of a Qudrant edge Orifice plate and state its advantages. |  |
| b) | Compare the performance of Thermocouple and RTD. |  |
| c) | Define Reynolds number and state the cut off limit for turbulent and laminar flow. |  |
| d) | Explain the reason why V to I and I to V converters are used in instrumentation. |  |
| e) | State one advantage and disadvantage of magnetic flow meter. |  |
| f) | Explain Coriollis effect in brief. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of vortex swirl and vortex shedding flow meters with the help of neat diagrams. |  |
| b) | Explain the working of turbine flow meter with the help of a diagram. |  |
| c) | Explain the working of any one type of ultrasonic flow meter with the help of a diagram. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and characteristics of RTD. Explain why platinum is a preferred material in RTD. |  |
| b) | Draw a diagram of liquid in metal thermometer and explain the construction and working. |  |
| c) | Explain the working of an optical pyrometer with the help of a neat diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a P to I converter with the help of a neat diagram. |  |
| b) | Explain the working of a McLeod gauge with the help of a diagram. |  |
| c) | Discuss angularity and multiplication adjustments in C type bourdon tube. Explain how back lash is minimized. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a high pressure sight glass level measurement system with the help of a diagram. |  |
| b) | Draw the diagram of a torque tube type liquid level detector and explain the working. |  |
| c) | Explain the working of a Capacitive level indicator with the help of a diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss any two calibration techniques used in flow meters. |  |
| b) | Draw the block diagram of closed loop system used for controlling temperature and explain the working. |  |
| c) | Explain the working of inclined plane manometer with the help of a diagram. |  |

***********